UNIT III NETWORK COMMUNICATION

Network Layer: Introduction - Network Layer Characteristics - IPv4 Packet - IPv6

Packet - Host Routing - Introduction to Routing

Address Resolution: Introduction - MAC and IP – ARP - IPv6 Neighbor Discovery

Basic Router Configuration: Introduction - Configure Initial Router Settings – Interfaces configuration - The Default Gateway configuration

3.1  Network Layer

By now you may have noticed that the modules in this course are progressing from the bottom up through the OSI model layers. At the network layer of the OSI model, we introduce you to communication protocols and routing protocols. Say you want to send an email to a friend who lives in another city, or even another country. This person is not on the same network as you. A simple switched network cannot get your message any further than the end of your own network. You need some help to keep this message moving along the path to your friend’s end device. To send an email (a video, or a file, etc.) to anyone who is not on your local network, you must have access to routers

3.1.1 Network Layer characteristics

Characteristics of IP

IP was designed as a protocol with low overhead. It provides only the functions that are necessary to deliver a packet from a source to a destination over an interconnected system of networks. The protocol was not designed to track and manage the flow of packets. These functions, if required, are performed by other protocols at other layers, primarily TCP at Layer 4.

These are the basic characteristics of IP:

- Connectionless - There is no connection with the destination established before sending data packets.

- Best Effort - IP is inherently unreliable because packet delivery is not guaranteed.

- Media Independent - Operation is independent of the medium (i.e., copper, fiber-optic, or wireless) carrying the data.

- Connectionless

- IP is connectionless, meaning that no dedicated end-to-end connection is created by IP before data is sent. Connectionless communication is conceptually similar to sending a letter to someone without notifying the recipient in advance. The figure summarizes this key point.

### Connectionless - Analogy

- a packet, consisting of an IP header and segment, is sent from a source on one network to a destination on another network

- A letter is sent.

- Letter

- Letter

- Mail box

- Connectionless data communications work on the same principle. As shown in the figure, IP requires no initial exchange of control information to establish an end-to-end connection before packets are forwarded.

### Connectionless - Network

Best Effort

IP also does not require additional fields in the header to maintain an established connection. This process greatly reduces the overhead of IP. However, with no pre-established end-to-end connection, senders are unaware whether destination devices are present and functional when sending packets, nor are they aware if the destination receives the packet, or if the destination device is able to access and read the packet.

The IP protocol does not guarantee that all packets that are delivered are, in fact, received. The figure illustrates the unreliable or best-effort delivery characteristic of the IP protocol.

The diagram shows a source on one network and a destination on another network. Between the two hosts is a cloud consisting of four routers in a mesh topology. Three IP packets leave the source host but only two arrive at the destination host. Text in the graphic reads: Packets are routed through the network quickly; Some Packets may be lost en route.

Packets are routed through the network quickly

Some Packets may be lost en route

IP Packet

IP Packet

IP Packet

IP Packet

IP Packet

As an unreliable network layer protocol, IP does not guarantee that all sent packets will be received. Other protocols manage the process of tracking packets and ensuring their delivery.

8.1.6 Media Independent

Unreliable means that IP does not have the capability to manage and recover from undelivered or corrupt packets. This is because while IP packets are sent with information about the location of delivery, they do not contain information that can be processed to inform the sender whether delivery was successful. Packets may arrive at the destination corrupted, out of sequence, or not at all. IP provides no capability for packet retransmissions if errors occur.

If out-of-order packets are delivered, or packets are missing, then applications using the data, or upper layer services, must resolve these issues. This allows IP to function very efficiently. In the TCP/IP protocol suite, reliability is the role of the TCP protocol at the transport layer.

IP operates independently of the media that carry the data at lower layers of the protocol stack. As shown in the figure, IP packets can be communicated as electronic signals over copper cable, as optical signals over fiber, or wirelessly as radio signals.

The diagram shows a network topology within a cloud with a packet traveling over various media types between two hosts. An IP packet is shown moving between a host and a router over a copper Ethernet connection. The first router is connected to second router via a copper serial connection. An IP packet is shown moving between the second router and a third router over an optical fiber connection. The third router is connected to a fourth router, which is a wireless router. An IP packet is shown moving between the fourth router and a host over a wireless connection.

IP Packet

IP Packet

IP Packet

Copper Ethernet

Copper Serial

Optical Fiber

Wireless

Copper Ethernet

IP packets can travel over different media.

The OSI data link layer is responsible for taking an IP packet and preparing it for transmission over the communications medium. This means that the delivery of IP packets is not limited to any particular medium.

There is, however, one major characteristic of the media that the network layer considers: the maximum size of the PDU that each medium can transport. This characteristic is referred to as the maximum transmission unit (MTU). Part of the control communication between the data link layer and the network layer is the establishment of a maximum size for the packet. The data link layer passes the MTU value up to the network layer. The network layer then determines how large packets can be.

In some cases, an intermediate device, usually a router, must split up an IPv4 packet when forwarding it from one medium to another medium with a smaller MTU. This process is called fragmenting the packet, or fragmentation. Fragmentation causes latency. IPv6 packets cannot be fragmented by the router.

3.1.2 IPV4 Packet

IPv4 Packet Header

IPv4 is one of the primary network layer communication protocols. The IPv4 packet header is used to ensure that this packet is delivered to its next stop on the way to its destination end device.

An IPv4 packet header consists of fields containing important information about the packet. These fields contain binary numbers which are examined by the Layer 3 process.

IPv4 Packet Header Fields

The binary values of each field identify various settings of the IP packet. Protocol header diagrams, which are read left to right, and top down, provide a visual to refer to when discussing protocol fields. The IP protocol header diagram in the figure identifies the fields of an IPv4 packet.

### Fields in the IPv4 Packet Header

names and bit length of fields in an IPv4 packet header

Byte 1

Byte 2

Byte 3

Byte 4

Version

Internet Header Length

DS

DSCP

ECN

Total Length

Identification

Flag

Fragment Offset

Time-to-Live

Protocol

Header Checksum

Source IP Address

Destination IP Address

...

20 Bytes

Significant fields in the IPv4 header include the following:

- Version - Contains a 4-bit binary value set to 0100 that identifies this as an IPv4 packet.

- Differentiated Services or DiffServ (DS) - Formerly called the type of service (ToS) field, the DS field is an 8-bit field used to determine the priority of each packet. The six most significant bits of the DiffServ field are the differentiated services code point (DSCP) bits and the last two bits are the explicit congestion notification (ECN) bits.

- Time to Live (TTL) - TTL contains an 8-bit binary value that is used to limit the lifetime of a packet. The source device of the IPv4 packet sets the initial TTL value. It is decreased by one each time the packet is processed by a router. If the TTL field decrements to zero, the router discards the packet and sends an Internet Control Message Protocol (ICMP) Time Exceeded message to the source IP address. Because the router decrements the TTL of each packet, the router must also recalculate the Header Checksum.

- Protocol - This field is used to identify the next level protocol. This 8-bit binary value indicates the data payload type that the packet is carrying, which enables the network layer to pass the data to the appropriate upper-layer protocol. Common values include ICMP (1), TCP (6), and UDP (17).

- Header Checksum - This is used to detect corruption in the IPv4 header.

- Source IPv4 Address - This contains a 32-bit binary value that represents the source IPv4 address of the packet. The source IPv4 address is always a unicast address.

- Destination IPv4 Address - This contains a 32-bit binary value that represents the destination IPv4 address of the packet. The destination IPv4 address is a unicast, multicast, or broadcast address.

The two most commonly referenced fields are the source and destination IP addresses. These fields identify where the packet is coming from and where it is going. Typically, these addresses do not change while travelling from the source to the destination.

The Internet Header Length (IHL), Total Length, and Header Checksum fields are used to identify and validate the packet.

Other fields are used to reorder a fragmented packet. Specifically, the IPv4 packet uses Identification, Flags, and Fragment Offset fields to keep track of the fragments. A router may have to fragment an IPv4 packet when forwarding it from one medium to another with a smaller MTU.

The Options and Padding fields are rarely used and are beyond the scope of this module.

3.1.3 IPV6 Packet

Limitations of IPv4

IPv4 is still in use today. This topic is about IPv6, which will eventually replace IPv4. To better understand why you need to know the IPv6 protocol, it helps to know the limitations of IPv4 and the advantages of IPv6.

Through the years, additional protocols and processes have been developed to address new challenges. However, even with changes, IPv4 still has three major issues:

- IPv4 address depletion - IPv4 has a limited number of unique public addresses available. Although there are approximately 4 billion IPv4 addresses, the increasing number of new IP-enabled devices, always-on connections, and the potential growth of less-developed regions have increased the need for more addresses.

- Lack of end-to-end connectivity - Network Address Translation (NAT) is a technology commonly implemented within IPv4 networks. NAT provides a way for multiple devices to share a single public IPv4 address. However, because the public IPv4 address is shared, the IPv4 address of an internal network host is hidden. This can be problematic for technologies that require end-to-end connectivity.

- Increased network complexity - While NAT has extended the lifespan of IPv4 it was only meant as a transition mechanism to IPv6. NAT in its various implementation creates additional complexity in the network, creating latency and making troubleshooting more difficult.

IPv6 Overview

In the early 1990s, the Internet Engineering Task Force (IETF) grew concerned about the issues with IPv4 and began to look for a replacement. This activity led to the development of IP version 6 (IPv6). IPv6 overcomes the limitations of IPv4 and is a powerful enhancement with features that better suit current and foreseeable network demands.

Improvements that IPv6 provides include the following:

- Increased address space - IPv6 addresses are based on 128-bit hierarchical addressing as opposed to IPv4 with 32 bits.

- Improved packet handling - The IPv6 header has been simplified with fewer fields.

- Eliminates the need for NAT - With such a large number of public IPv6 addresses, NAT between a private IPv4 address and a public IPv4 is not needed. This avoids some of the NAT-induced problems experienced by applications that require end-to-end connectivity.

The 32-bit IPv4 address space provides approximately 4,294,967,296 unique addresses. IPv6 address space provides 340,282,366,920,938,463,463,374,607,431,768,211,456, or 340 undecillion addresses. This is roughly equivalent to every grain of sand on Earth.

The figure provides a visual to compare the IPv4 and IPv6 address space.

### IPv4 and IPv6 Address Space Comparison

comparison of IPv4 and IPv6 address space

| Number Name | Scientific Notation | Number of Zeros |
| --- | --- | --- |
| 1 Thousand | 10^3 | 1,000 |
| 1 Million | 10^6 | 1,000,000 |
| 1 Billion | 10^9 | 1,000,000,000 |
| 1 Trillion | 10^12 | 1,000,000,000,000 |
| 1 Quadrillion | 10^15 | 1,000,000,000,000,000 |
| 1 Quintillion | 10^18 | 1,000,000,000,000,000,000 |
| 1 Sextillion | 10^21 | 1,000,000,000,000,000,000,000 |
| 1 Septillion | 10^24 | 1,000,000,000,000,000,000,000,000 |
| 1 Octillion | 10^27 | 1,000,000,000,000,000,000,000,000,000 |
| 1 Nonillion | 10^30 | 1,000,000,000,000,000,000,000,000,000,000 |
| 1 Decillion | 10^33 | 1,000,000,000,000,000,000,000,000,000,000,000 |
| 1 Undecillion | 10^36 | 1,000,000,000,000,000,000,000,000,000,000,000,000 |

IPv4 Packet Header Fields in the IPv6 Packet Header

One of the major design improvements of IPv6 over IPv4 is the simplified IPv6 header.

For example, the IPv4 header consists of a variable length header of 20 octets (up to 60 bytes if the Options field is used) and 12 basic header fields, not including the Options field and Padding field.

For IPv6, some fields have remained the same, some fields have changed names and positions, and some IPv4 fields are no longer required, as highlighted in the figure.

### IPv4 Packet Header

The diagram shows an IPv4 packet header and indicates which fields kept the same name, which fields changed names and position, and which fields were not kept in IPv6. The fields that kept the same name are: version, source address, and destination address. The fields that changed names and position are: type of service, total length, time-to-live, and protocol. The fields that were not kept in IPv6 are: IHL, identification, flags, fragment offset, header checksum, options, and padding.

Byte 1

Byte 2

Byte 3

Byte 4

Version

IHL

Type of Service

Options

Padding

Total Length

Destination Address

Legend

- Fields name kept from IPv4 to IPv6

Identification

Flags

Fragment Offset

Time-to-Live

Protocol

Header Checksum

Source Address

- Name and position changed in IPv6

- Fields no longer required in IPv6.

20 Bytes

The figure shows IPv4 packet header fields that were kept, moved, changed, as well as those that were not kept in the IPv6 packet header.

In contrast, the simplified IPv6 header shown the next figure consists of a fixed length header of 40 octets (largely due to the length of the source and destination IPv6 addresses).

The IPv6 simplified header allows for more efficient processing of IPv6 headers.

### IPv6 Packet Header

The diagram shows an IPv6 packet header and indicates which fields kept the same name from IPv4 to IPv6, which fields changed names and position in IPv6, which fields were not kept in IPv6, and new fields in IPv6. The field names that were kept the same are: version, source IP address, and destination IP address. The fields that changed names and position in IPv6 are: traffic class, payload length, next header, and hop limit. The field that is NEW to IPv6 is flow label.

Byte 1

Byte 2

Byte 3

Byte 4

Version

Traffic Class

Flow Label

Next Header

Payload Length

Hop Limit

Destination IP Address

Source IP Address

- Fields name kept from IPv4 to IPv6

- Name and position changed in IPv6

- Fields no longer required in IPv6

Legend

40 bytes

3.1.4 Host Routing

A Router is a process of selecting path along which the data can be transferred from

source to the destination. Routing is performed by a special device known as a router.

 A Router works at the network layer in the OSI model and internet layer in TCP/IP

model

 A router is a networking device that forwards the packet based on the information

available in the packet header and forwarding table.

 The routing algorithms are used for routing the packets. The routing algorithm is

nothing but a software responsible for deciding the optimal path through which packet

can be transmitted.

 The routing protocols use the metric to determine the best path for the packet

delivery. The metric is the standard of measurement such as hop count, bandwidth,

delay, current load on the path, etc. used by the routing algorithm to determine the

optimal path to the destination.

 The routing algorithm initializes and maintains the routing table for the process of

path determination.

The most common metric values are given below:

 Hop count: Hop count is defined as a metric that specifies the number of passes

through internetworking devices such as a router, a packet must travel in a route to

move from source to the destination. If the routing protocol considers the hop as a

primary metric value, then the path with the least hop count will be considered as the

best path to move from source to the destination.

 Delay: It is a time taken by the router to process, queue and transmit a datagram to an

interface. The protocols use this metric to determine the delay values for all the links

along the path end-to-end. The path having the lowest delay value will be considered

as the best path.

 Bandwidth: The capacity of the link is known as a bandwidth of the link. The

bandwidth is measured in terms of bits per second. The link that has a higher transfer

rate like gigabit is preferred over the link that has the lower capacity like 56 kb. The

protocol will determine the bandwidth capacity for all the links along the path, and the

overall higher bandwidth will be considered as the best route.

 Load: Load refers to the degree to which the network resource such as a router or

network link is busy. A Load can be calculated in a variety of ways such as CPU

utilization, packets processed per second. If the traffic increases, then the load value

will also be increased. The load value changes with respect to the change in the

traffic.

 Reliability: Reliability is a metric factor may be composed of a fixed value. It

depends on the network links, and its value is measured dynamically. Some networks

go down more often than others. After network failure, some network links repaired

more easily than other network links. Any reliability factor can be considered for the

assignment of reliability ratings, which are generally numeric values assigned by the

system administrator.

Types of Routing

Routing can be classified into three categories:

 Static Routing

 Default Routing

 Dynamic Routing

Static Routing

 Static Routing is also known as Nonadaptive Routing.

 It is a technique in which the administrator manually adds the routes in a routing

table.

 A Router can send the packets for the destination along the route defined by the

administrator.

 In this technique, routing decisions are not made based on the condition or topology

of the networks

Advantages Of Static Routing

Following are the advantages of Static Routing:

 No Overhead: It has ho overhead on the CPU usage of the router. Therefore, the

cheaper router can be used to obtain static routing.

 Bandwidth: It has not bandwidth usage between the routers.

 Security: It provides security as the system administrator is allowed only to have

control over the routing to a particular network.

Disadvantages of Static Routing:

Following are the disadvantages of Static Routing:

 For a large network, it becomes a very difficult task to add each route manually to the

routing table.

 The system administrator should have a good knowledge of a topology as he has to

add each route manually.

Default Routing

 Default Routing is a technique in which a router is configured to send all the packets

to the same hop device, and it doesn&#39;t matter whether it belongs to a particular

network or not. A Packet is transmitted to the device for which it is configured.

Default Routing is used when networks deal with the single exit point.

 It is also useful when the bulk of transmission networks have to transmit the data to

the same hp device.

 When a specific route is mentioned in the routing table, the router will choose the

specific route rather than the default route. The default route is chosen only when a

specific route is not mentioned in the routing table.

Dynamic Routing

 It is also known as Adaptive Routing.

 It is a technique in which a router adds a new route in the routing table for each packet

in response to the changes in the condition or topology of the network.

 Dynamic protocols are used to discover the new routes to reach the destination.

 In Dynamic Routing, RIP and OSPF are the protocols used to discover the new routes.

 If any route goes down, then the automatic adjustment will be made to reach the

destination.

The Dynamic protocol should have the following features:

 All the routers must have the same dynamic routing protocol in order to exchange the

routes.

 If the router discovers any change in the condition or topology, then router broadcast

this information to all other routers.

Advantages of Dynamic Routing:

 It is easier to configure.

 It is more effective in selecting the best route in response to the changes in the

condition or topology.

Disadvantages of Dynamic Routing:

 It is more expensive in terms of CPU and bandwidth usage.

 It is less secure as compared to default and static routing.

4.2 Unicast routing

Unicast – Unicast means the transmission from a single sender to a single receiver. It is a

point-to-point communication between sender and receiver. There are various unicast

protocols such as TCP, HTTP, etc.

 TCP is the most commonly used unicast protocol. It is a connection-oriented protocol

that relies on acknowledgement from the receiver side.

 HTTP stands for HyperText Transfer Protocol. It is an object-oriented protocol for

communication.

There are three major protocols for unicast routing:

1. Distance Vector Routing

2. Link State Routing

3. Path-Vector Routing

4.2.1 Distance Vector Routing

Distance vector routing algorithm is also called as Bellman-Ford algorithm or Ford

Fulkerson algorithm as this algorithm is used to find the shortest route from one node to another node in the network.

The routing protocol is used to calculate the best route from source to destination based on the distance or hops as its primary metric to define an optimal path. The distance vector refers to the distance to the neighbor nodes, where routing defines the routes to the established node.

The Distance Vector routing algorithm(DVR) shares the information of the routing table with the other routers in the network .

3.2 Address Resolution

Hosts and routers both create routing tables to ensure that they can send and receive data across networks. So how does this information get created in a routing table? As a network administrator, you could enter these MAC and IP addresses manually. But that would take a lot of time and the likelihood of making a few mistakes is great. Are you thinking that there must be some way that this could be done automatically, by the hosts and routers themselves? Of course, you are correct! And even though it is automatic, you must still understand how this works, because you may have to troubleshoot a problem, or worse, your network could be attacked by a threat actor.

3.2.1 MAC and IP

Destination on Same Network

Sometimes a host must send a message, but it only knows the IP address of the destination device. The host needs to know the MAC address of that device, but how can it be discovered? That is where address resolution becomes critical.

There are two primary addresses assigned to a device on an Ethernet LAN:

- Physical address (the MAC address) - Used for NIC to NIC communications on the same Ethernet network.

- Logical address (the IP address) - Used to send the packet from the source device to the destination device. The destination IP address may be on the same IP network as the source or it may be on a remote network.

Layer 2 physical addresses (i.e., Ethernet MAC addresses) are used to deliver the data link frame with the encapsulated IP packet from one NIC to another NIC that is on the same network. If the destination IP address is on the same network, the destination MAC address will be that of the destination device.

Consider the following example using simplified MAC address representations.

The image is a network diagram with PC 1 at IP 192.168.10.10/24 with simplified MAC aa-aa-aa, connected to a switch at IP 192.168.10.0/24, connected to PC 2 at IP 192.168.10.11/24 with simplified MAC 55-55-55. Below the diagram are four boxes reading from left to right: Destination MAC 55-55-55, Source MAC aa-aa-aa, Source IPv4 192.168.10.10, and Destination IPv4 192.168.10.11.

| Destination MAC | Source MAC | Source IPv4 | Destination IPv4 |
| --- | --- | --- | --- |
| 55-55-55 | aa-aa-aa | 192.168.10.10 | 192.168.10.11 |

55-55-55

aa-aa-aa

PC 1

PC 2

192.168.10.10/24

192.168.10.0/24

192.168.10.11/24

In this example, PC1 wants to send a packet to PC2. The figure displays the Layer 2 destination and source MAC addresses and the Layer 3 IPv4 addressing that would be included in the packet sent from PC1.

The Layer 2 Ethernet frame contains the following:

- Destination MAC address - This is the simplified MAC address of PC2, 55-55-55.

- Source MAC address - This is the simplified MAC address of the Ethernet NIC on PC1, aa-aa-aa.

The Layer 3 IP packet contains the following:

- Source IPv4 address - This is the IPv4 address of PC1, 192.168.10.10.

- Destination IPv4 address - This is the IPv4 address of PC2, 192.168.10.11.

9.1.2 Destination on Remote Network

When the destination IP address (IPv4 or IPv6) is on a remote network, the destination MAC address will be the address of the host default gateway (i.e., the router interface).

Consider the following example using a simplified MAC address representation.

The image is a network diagram showing the source and destination MAC and IPv4 addressing for the first hop when information is sent from a PC on one network to a destination on a remote network. The description that follows are the names, connections, and addressing of devices from left to right. PC 1 is connected to a switch which is connected to router R1 interface G0/0/0 on network 192.168.10.0/24. PC 1 has an IP of 192.168.10.10 and MAC of aa-aa-aa. The R1 G0/0/0 interface has an IP of 192.168.10.1 and MAC of bb-bb-bb. R1 has a G0/0/1 interface connected to router R2 interface G0/0/1 on network 209.165.200.224/30. The R1 G0/0/1 interface has an IP of 209.165.200.225 and MAC cc-cc-cc. The R2 G0/0/1 interface has an IP of 209.165.200.226 and MAC dd-dd-dd. R2 has a G0/0/0 interface connected to a switch connected to PC 2 on network 10.1.1.0/24. The R2 G0/0/0 interface has an IP of 10.1.1.1 and MAC ee-ee-ee. PC 2 has an IP of 10.10.10.10 and MAC 55-55-55. R2 also has a connection at the top leading to the Internet cloud. Below the diagram under network 192.168.10.0/24 are four boxes reading from left to right: Destination MAC bb-bb-bb, Source MAC aa-aa-aa, Source IPv4 192.168.10.10, and Destination IPv4 10.1.1.10.

R1R2PC 2 PC 1.10.1.225.226.1.10G0/0/1G0/0/1G0/0/0G0/0/0

| Destination MAC | Source MAC | Source IPv4 | Destination IPv4 |
| --- | --- | --- | --- |
| bb-bb-bb | aa-aa-aa | 192.168.10.10 | 10.1.1.10 |

55-55-55

aa-aa-aa

192.168.10.0/24

209.165.200.224/30

10.1.1.0/24

Internet

bb-bb-bb

cc-cc-cc

dd-dd-dd

ee-ee-ee

In this example, PC1 wants to send a packet to PC2. PC2 is located on remote network. Because the destination IPv4 address is not on the same local network as PC1, the destination MAC address is that of the local default gateway on the router.

Routers examine the destination IPv4 address to determine the best path to forward the IPv4 packet. When the router receives the Ethernet frame, it de-encapsulates the Layer 2 information. Using the destination IPv4 address, it determines the next-hop device, and then encapsulates the IPv4 packet in a new data link frame for the outgoing interface.

In our example, R1 would now encapsulate the packet with new Layer 2 address information as shown in the figure.

The image is a network diagram showing the source and destination MAC and IPv4 addressing when information is passed between two routers as it is sent from a PC on one network to a destination on a remote network. The description that follows are the names, connections, and addressing of devices from left to right. PC 1 is connected to a switch which is connected to router R1 interface G0/0/0 on network 192.168.10.0/24. PC 1 has an IP of 192.168.10.10 and MAC of aa-aa-aa. The R1 G0/0/0 interface has an IP of 192.168.10.1 and MAC of bb-bb-bb. R1 has a G0/0/1 interface connected to router R2 interface G0/0/1 on network 209.165.200.224/30. The R1 G0/0/1 interface has an IP of 209.165.200.225 and MAC cc-cc-cc. The R2 G0/0/1 interface has an IP of 209.165.200.226 and MAC dd-dd-dd. R2 has a G0/0/0 interface connected to a switch connected to PC 2 on network 10.1.1.0/24. The R2 G0/0/0 interface has an IP of 10.1.1.1 and MAC ee-ee-ee. PC 2 has an IP of 10.10.10.10 and MAC 55-55-55. R2 also has a connection at the top leading to the Internet cloud. Below the diagram under network 209.165.200.224/30 are four boxes reading from left to right: Destination MAC dd-dd-dd, Source MAC cc-cc-cc, Source IPv4 192.168.10.10, and Destination IPv4 10.1.1.10.

R1R2PC 2 PC 1.10.1.225.226.1.10G0/0/1G0/0/1G0/0/0G0/0/0

| Destination MAC | Source MAC | Source IPv4 | Destination IPv4 |
| --- | --- | --- | --- |
| dd-dd-dd | cc-cc-cc | 192.168.10.10 | 10.1.1.10 |

55-55-55

aa-aa-aa

192.168.10.0/24

209.165.200.224/30

10.1.1.0/24

Internet

bb-bb-bb

cc-cc-cc

dd-dd-dd

ee-ee-ee

The new destination MAC address would be that of the R2 G0/0/1 interface and the new source MAC address would be that of the R1 G0/0/1 interface.

Along each link in a path, an IP packet is encapsulated in a frame. The frame is specific to the data link technology that is associated with that link, such as Ethernet. If the next-hop device is the final destination, the destination MAC address will be that of the device Ethernet NIC, as shown in the figure.

The image is a network diagram showing the source and destination MAC and IPv4 addressing when information exits a router to the final destination as it is sent from a PC on one network to a destination on a remote network. The description that follows are the names, connections, and addressing of devices from left to right. PC 1 is connected to a switch which is connected to router R1 interface G0/0/0 on network 192.168.10.0/24. PC 1 has an IP of 192.168.10.10 and MAC of aa-aa-aa. The R1 G0/0/0 interface has an IP of 192.168.10.1 and MAC of bb-bb-bb. R1 has a G0/0/1 interface connected to router R2 interface G0/0/1 on network 209.165.200.224/30. The R1 G0/0/1 interface has an IP of 209.165.200.225 and MAC cc-cc-cc. The R2 G0/0/1 interface has an IP of 209.165.200.226 and MAC dd-dd-dd. R2 has a G0/0/0 interface connected to a switch connected to PC 2 on network 10.1.1.0/24. The R2 G0/0/0 interface has an IP of 10.1.1.1 and MAC ee-ee-ee. PC 2 has an IP of 10.10.10.10 and MAC 55-55-55. R2 also has a connection at the top leading to the Internet cloud. Below the diagram under network 10.1.1.0/24 are four boxes reading from left to right: Destination MAC 55-55-55, Source MAC ee-ee-ee, Source IPv4 192.168.10.10, and Destination IPv4 10.1.1.10.

R1R2PC 2 PC 1.10.1.225.226.1.10G0/0/1G0/0/1G0/0/0G0/0/0

| Destination MAC | Source MAC | Source IPv4 | Destination IPv4 |
| --- | --- | --- | --- |
| 55-55-55 | ee-ee-ee | 192.168.10.10 | 10.1.1.10 |

55-55-55

aa-aa-aa

192.168.10.0/24

209.165.200.224/30

10.1.1.0/24

Internet

bb-bb-bb

cc-cc-cc

dd-dd-dd

ee-ee-ee

How are the IP addresses of the IP packets in a data flow associated with the MAC addresses on each link along the path to the destination? For IPv4 packets, this is done through a process called Address Resolution Protocol (ARP). For IPv6 packets, the process is ICMPv6 Neighbor Discovery (ND).

3.2.2 ARP

ARP Overview

If your network is using the IPv4 communications protocol, the Address Resolution Protocol, or ARP, is what you need to map IPv4 addresses to MAC addresses. This topic explains how ARP works.

Every IP device on an Ethernet network has a unique Ethernet MAC address. When a device sends an Ethernet Layer 2 frame, it contains these two addresses:

- Destination MAC address - The Ethernet MAC address of the destination device on the same local network segment. If the destination host is on another network, then the destination address in the frame would be that of the default gateway (i.e., router).

- Source MAC address - The MAC address of the Ethernet NIC on the source host.

The figure illustrates the problem when sending a frame to another host on the same segment on an IPv4 network.

Four hosts, H1, H2, H3, and H4, are connected to the same switch. H1 has an IP of 192.168.1.5/24, H2 has an IP of 192.168.1.6/24, H3 has an IP of 192.168.1.8/24, and H4 has an IP of 192.168.1.7/24. H1 has a callout that reads: I need to send information to 192.168.1.7, but I only have the IP address. I dont know the MAC address of the device that has that IP.

H2HIH4H3192.168.1.5/24192.168.1.6/24192.168.1.8/24192.168.1.7/24

I need to send information to 192.168.1.7, but I only have the IP address. I don't know the MAC address of the device that has that IP.

To send a packet to another host on the same local IPv4 network, a host must know the IPv4 address and the MAC address of the destination device. Device destination IPv4 addresses are either known or resolved by device name. However, MAC addresses must be discovered.

A device uses Address Resolution Protocol (ARP) to determine the destination MAC address of a local device when it knows its IPv4 address.

ARP provides two basic functions:

- Resolving IPv4 addresses to MAC addresses

- Maintaining a table of IPv4 to MAC address mappings

9.2.2 ARP Functions

When a packet is sent to the data link layer to be encapsulated into an Ethernet frame, the device refers to a table in its memory to find the MAC address that is mapped to the IPv4 address. This table is stored temporarily in RAM memory and called the ARP table or the ARP cache.

The sending device will search its ARP table for a destination IPv4 address and a corresponding MAC address.

- If the packet’s destination IPv4 address is on the same network as the source IPv4 address, the device will search the ARP table for the destination IPv4 address.

- If the destination IPv4 address is on a different network than the source IPv4 address, the device will search the ARP table for the IPv4 address of the default gateway.

In both cases, the search is for an IPv4 address and a corresponding MAC address for the device.

Each entry, or row, of the ARP table binds an IPv4 address with a MAC address. We call the relationship between the two values a map. This simply means that you can locate an IPv4 address in the table and discover the corresponding MAC address. The ARP table temporarily saves (caches) the mapping for the devices on the LAN.

If the device locates the IPv4 address, its corresponding MAC address is used as the destination MAC address in the frame. If there is no entry is found, then the device sends an ARP request.

Click Play in the figure to see an animation of the ARP function.

This animation illustrates how a host will use A R P to discover the MAC address of a known I P address. Host H1 needs to send some information to a host with IP address 192 dot 168 dot 1 dot 7. However, H1 does not have the MAC address for that address. Therefore, it sends an A R P request to I P address 192.168.1.7. All hosts on the network will receive the A R P request. However, only host H4 with IP address 192.168.1.7 will send an A R P reply containing its MAC address. Then H1 can send an envelope to the switch that goes directly to H4.

I must send out an ARP request to learn the MAC address of the host with the IP address of 192.168.1.7.

This isn't me.

This isn't me.

This is me, I will send back my MAC address.

Now I have the MAC. I can forward my information.

Thanks. I got it.

9.2.3 Video - ARP Request

An ARP request is sent when a device needs to determine the MAC address that is associated with an IPv4 address, and it does not have an entry for the IPv4 address in its ARP table.

ARP messages are encapsulated directly within an Ethernet frame. There is no IPv4 header. The ARP request is encapsulated in an Ethernet frame using the following header information:

- Destination MAC address - This is a broadcast address FF-FF-FF-FF-FF-FF requiring all Ethernet NICs on the LAN to accept and process the ARP request.

- Source MAC address - This is MAC address of the sender of the ARP request.

- Type - ARP messages have a type field of 0x806. This informs the receiving NIC that the data portion of the frame needs to be passed to the ARP process.

Because ARP requests are broadcasts, they are flooded out all ports by the switch, except the receiving port. All Ethernet NICs on the LAN process broadcasts and must deliver the ARP request to its operating system for processing. Every device must process the ARP request to see if the target IPv4 address matches its own. A router will not forward broadcasts out other interfaces.

Only one device on the LAN will have an IPv4 address that matches the target IPv4 address in the ARP request. All other devices will not reply.

Click Play in the figure to view a demonstration of an ARP request for a destination IPv4 address that is on the local network.

9.2.4 Video - ARP Operation - ARP Reply

Only the device with the target IPv4 address associated with the ARP request will respond with an ARP reply. The ARP reply is encapsulated in an Ethernet frame using the following header information:

- Destination MAC address - This is the MAC address of the sender of the ARP request.

- Source MAC address - This is the MAC address of the sender of the ARP reply.

- Type - ARP messages have a type field of 0x806. This informs the receiving NIC that the data portion of the frame needs to be passed to the ARP process.

Only the device that originally sent the ARP request will receive the unicast ARP reply. After the ARP reply is received, the device will add the IPv4 address and the corresponding MAC address to its ARP table. Packets destined for that IPv4 address can now be encapsulated in frames using its corresponding MAC address.

If no device responds to the ARP request, the packet is dropped because a frame cannot be created.

Entries in the ARP table are time stamped. If a device does not receive a frame from a particular device before the timestamp expires, the entry for this device is removed from the ARP table.

Additionally, static map entries can be entered in an ARP table, but this is rarely done. Static ARP table entries do not expire over time and must be manually removed.

Note: IPv6 uses a similar process to ARP for IPv4, known as ICMPv6 Neighbor Discovery (ND). IPv6 uses neighbor solicitation and neighbor advertisement messages, similar to IPv4 ARP requests and ARP replies.

Click Play in the figure to view a demonstration of an ARP reply.

9.2.5 Video - ARP Role in Remote Communications

When the destination IPv4 address is not on the same network as the source IPv4 address, the source device needs to send the frame to its default gateway. This is the interface of the local router. Whenever a source device has a packet with an IPv4 address on another network, it will encapsulate that packet in a frame using the destination MAC address of the router.

The IPv4 address of the default gateway is stored in the IPv4 configuration of the hosts. When a host creates a packet for a destination, it compares the destination IPv4 address and its own IPv4 address to determine if the two IPv4 addresses are located on the same Layer 3 network. If the destination host is not on its same network, the source checks its ARP table for an entry with the IPv4 address of the default gateway. If there is not an entry, it uses the ARP process to determine a MAC address of the default gateway.

Click Play to view a demonstration of an ARP request and ARP reply associated with the default gateway.

9.2.6 Removing Entries from an ARP Table

For each device, an ARP cache timer removes ARP entries that have not been used for a specified period of time. The times differ depending on the operating system of the device. For example, newer Windows operating systems store ARP table entries between 15 and 45 seconds, as illustrated in the figure.

ACBR1S1G0/0/0

192.168.1.110/24
MAC 00-0A

192.168.1.120/24
MAC 00-0B

192.168.1.50/24
MAC 00-0C

192.168.1.1/24
MAC 00-0D

I will remove this ARP entry if I have not used it within 15 to 45 seconds.

Internet

Note: MAC addresses are shortened for demonstration purposes.

| PC A's ARP Cache |
| --- |
| IPv4 Address | MAC Address |
| 192.168.1.1 | 00:0D |

Commands may also be used to manually remove some or all of the entries in the ARP table. After an entry has been removed, the process for sending an ARP request and receiving an ARP reply must occur again to enter the map in the ARP table.

9.2.7 ARP Tables on Networking Devices

On a Cisco router, the show ip arp command is used to display the ARP table, as shown in the figure.

R1# show ip arp

Protocol  Address          Age (min)  Hardware Addr   Type   Interface

Internet  192.168.10.1            -   a0e0.af0d.e140  ARPA   GigabitEthernet0/0/0

Internet  209.165.200.225         -   a0e0.af0d.e141  ARPA   GigabitEthernet0/0/1

Internet  209.165.200.226         1   a03d.6fe1.9d91  ARPA   GigabitEthernet0/0/1

R1#

On a Windows 10 PC, the arp -a command is used to display the ARP table, as shown in the figure.

C:\Users\PC> arp -a

Interface: 192.168.1.124 --- 0x10

Internet Address      Physical Address      Type

192.168.1.1           c8-d7-19-cc-a0-86     dynamic

192.168.1.101         08-3e-0c-f5-f7-77     dynamic

192.168.1.110         08-3e-0c-f5-f7-56     dynamic

192.168.1.112         ac-b3-13-4a-bd-d0     dynamic

192.168.1.117         08-3e-0c-f5-f7-5c     dynamic

192.168.1.126         24-77-03-45-5d-c4     dynamic

192.168.1.146         94-57-a5-0c-5b-02     dynamic

192.168.1.255         ff-ff-ff-ff-ff-ff     static

224.0.0.22            01-00-5e-00-00-16     static

224.0.0.251           01-00-5e-00-00-fb     static

239.255.255.250       01-00-5e-7f-ff-fa     static

255.255.255.255       ff-ff-ff-ff-ff-ff     static

C:\Users\PC>

9.2.8 ARP Issues - ARP Broadcasts and ARP Spoofing

As a broadcast frame, an ARP request is received and processed by every device on the local network. On a typical business network, these broadcasts would probably have minimal impact on network performance. However, if a large number of devices were to be powered up and all start accessing network services at the same time, there could be some reduction in performance for a short period of time, as shown in the figure. After the devices send out the initial ARP broadcasts and have learned the necessary MAC addresses, any impact on the network will be minimized.

The diagram shows seven devices on shared media (multiple access) all powered on at the same time. A text box reads: ARP broadcasts can flood the local media.

All devices powered on at the same time

ARP broadcasts can flood the local media.

Shared Media (multiple access)

In some cases, the use of ARP can lead to a potential security risk. A threat actor can use ARP spoofing to perform an ARP poisoning attack. This is a technique used by a threat actor to reply to an ARP request for an IPv4 address that belongs to another device, such as the default gateway, as shown in the figure. The threat actor sends an ARP reply with its own MAC address. The receiver of the ARP reply will add the wrong MAC address to its ARP table and send these packets to the threat actor.

Enterprise level switches include mitigation techniques known as dynamic ARP inspection (DAI). DAI is beyond the scope of this course.

The image is a network diagram showing two hosts, Host A with IP 192.168.1.110/24 and MAC 00-0A and Host B with IP 192.168.1.120/24 and MAC 00-0B, connected to switch S1 which is connected to router R1 at port G0/0/0 (the default gateway with IP 192.168.1.1/24 and MAC 00-0D) which is connected to the Internet cloud. Also connected to S1 is a threat actor at host C with IP 192.168.1.50/24 and MAC 00-0C. Host A has a callout that reads: ARP Request: I need the MAC address of default gateway, 192.168.1.1. The threat actor host C has a callout that reads: I will send an ARP reply and pretend to be the default gateway! Note: MAC addresses are shortened for demonstration purposes.

ABS1R1CA192.168.1.110/24MAC 00-0A192.168.1.120/24MAC 00-0B192.168.1.50/24MAC 00-0C192.168.1.1/24MAC 00-0DG0/0/0

Network

Note: MAC addresses are shortened for demonstration purposes.

I will send an ARP reply and pretend to be the default gateway!

ARP Request: I need the MAC address of default gateway, 192.168.1.1

3.2.3 	IPV5 Neighbour Discovery

IPv6 Neighbor Discovery Messages

IPv6 Neighbor Discovery protocol is sometimes referred to as ND or NDP. In this course, we will refer to it as ND. ND provides address resolution, router discovery, and redirection services for IPv6 using ICMPv6. ICMPv6 ND uses five ICMPv6 messages to perform these services:

- Neighbor Solicitation messages

- Neighbor Advertisement messages

- Router Solicitation messages

- Router Advertisement messages

- Redirect Message

Neighbor Solicitation and Neighbor Advertisement messages are used for device-to-device messaging such as address resolution (similar to ARP for IPv4). Devices include both host computers and routers.

:11:102001:db8:acad:1::/64PC1PC2

Device-to-Device
Messaging

Router Solicitation and Router Advertisement messages are for messaging between devices and routers. Typically router discovery is used for dynamic address allocation and stateless address autoconfiguration (SLAAC).

:12001:db8:acad:1::/64PC1R1

Device-Router
Messaging

Note: The fifth ICMPv6 ND message is a redirect message which is used for better next-hop selection. This is beyond the scope of this course.

IPv6 ND is defined in the IETF RFC 4861.

9.3.3 IPv6 Neighbor Discovery - Address Resolution

Much like ARP for IPv4, IPv6 devices use IPv6 ND to determine the MAC address of a device that has a a known IPv6 address.

ICMPv6 Neighbor Solicitation and Neighbor Advertisement messages are used for MAC address resolution. This is similar to ARP Requests and ARP Replies used by ARP for IPv4. For example, assume PC1 wants to ping PC2 at IPv6 address 2001:db8:acad::11. To determine the MAC address for the known IPv6 address, PC1 sends an ICMPv6 Neighbor Solicitation message as illustrated in the figure.

ICMPv6 Neighbor Solicitation messages are sent using special Ethernet and IPv6 multicast addresses. This allows the Ethernet NIC of the receiving device to determine whether the Neighbor Solicitation message is for itself without having to send it to the operating system for processing.

PC2 replies to the request with an ICMPv6 Neighbor Advertisement message which includes its MAC address.

9.3.4 Packet Tracer - IPv6 Neighbor Discovery

In order for a device to communicate with another device, the MAC address of the destination device must be known. With IPv6, a process called Neighbor Discovery is responsible for determining the destination MAC address. You will gather PDU information in simulation mode to better understand the process. There is no Packet Tracer scoring for this activity.

descriptionIPv6 Neighbor Discoveryfile_downloadIPv6 Neighbor Discovery

3.3 Basic Router Configuration

3.3.1 Basic Router Configuration Steps

The following tasks should be completed when configuring initial settings on a router.

- Configure the device name.

Router(config)# hostname hostname

2. Secure privileged EXEC mode.

Router(config)# enable secret password

3. Secure user EXEC mode.

Router(config)# line console 0  
  Router(config-line)# password password  
  Router(config-line)# login

4. Secure remote Telnet / SSH access.

Router(config-line)# line vty 0 4  
  Router(config-line)# password   password  
  Router(config-line)# login  
  Router(config-line)# transport input {    ssh   | telnet}

5. Secure all passwords in the config file.

Router(config-line)# exit  
  Router(config)# service password-encryption

6. Provide legal notification.

Router(config)# banner motd delimiter message delimiter

7. Save the configuration.

Router(config)# end  
  Router# copy running-config startup-config

10.1.2 Basic Router Configuration Example

In this example, router R1 in the topology diagram will be configured with initial settings.

The figure shows a network topology diagram with two PCs, two switches, two routers, and an internet cloud. From left to right PC 1 connects to a switch which connects to R1 which connects to R2 which connects to a second switch, which connects to PC2. PC1 is on the 192.168.10.0/24 IPv4 network and has IPv4 address 192.168.10.10. PC1 also connects to the 2001.db8:acad:10::/64 IPv6 network and has IPv6 address 2001:db8:acad:10::10. Router R1 G0/0/0 interface is on the same network as PC1. The IPv4 and IPv6 address of the G0/0/0 interface of R1 is 192.168.10.1 and 2001:db8:acad:10::1. The IPv4 network connecting R1 and R2 is 209.165.200.224/30. The IPv6 network connecting R1 and R2 is 2001:db8:feed:224::/64. R1 connects to R2 over interface G0/0/1 which has IPv4 address 209.165.200. 225 and IPv6 address 2001:db8:feed:224::1. The IP addresses for R2 on the shared network with R1 are 209.165.200. 226 and 2001:db8:feed:224::2. PC2 and R2 are connected on IPv4 network 10.1.1.0/24 and IPv6 network 2001:db8:cafe:1::/64. PC1 has IPv4 address 10.1.1.10 and IPv6 address 2001:db8:cafe::10. R2 has IPv4 address 10.1.1.1 and IPv6 address 2001:db8:cafe::1.

.10::10192.168.10.0/24.1::1G0/0/0G0/0/1.225::1.226::2.1::1209.165.200.224/3010.1.1.0/24.10::102001:db8:acad:10::/642001:db8:feed:224::/642001:db8:cafe:1::/64PC1PC2R1R2

Internet

To configure the device name for R1, use the following commands.

Router> enable

Router# configure terminal

Enter configuration commands, one per line.

End with CNTL/Z.

Router(config)# hostname R1

R1(config)#

Note: Notice how the router prompt now displays the router hostname.

All router access should be secured. Privileged EXEC mode provides the user with complete access to the device and its configuration. Therefore, it is the most important mode to secure.

The following commands secure privileged EXEC mode and user EXEC mode, enable Telnet and SSH remote access, and encrypt all plaintext (i.e., user EXEC and VTY line) passwords.

R1(config)# enable secret class

R1(config)#

R1(config)# line console 0

R1(config-line)# password cisco

R1(config-line)# login

R1(config-line)# exit

R1(config)#

R1(config)# line vty 0 4

R1(config-line)# password cisco

R1(config-line)# login

R1(config-line)# transport input ssh telnet

R1(config-line)# exit

R1(config)#

R1(config)# service password-encryption

R1(config)#

The legal notification warns users that the device should only be accessed by permitted users. Legal notification is configured as follows.

R1(config)# banner motd #

Enter TEXT message. End with a new line and the #

***********************************************

WARNING: Unauthorized access is prohibited!

***********************************************

#

R1(config)#

If the previous commands were configured and the router accidently lost power, all configured commands would be lost. For this reason, it is important to save the configuration when changes are implemented. The following command saves the configuration to NVRAM.

R1# copy running-config startup-config

Destination filename [startup-config]?

Building configuration...

[OK]

R1#

Syntax Checker - Configure Initial Router Settings

Use this syntax checker to practice configuring the initial settings on a router.

- Configure the device name.

- Secure the privileged EXEC mode.

- Secure and enable remote SSH and Telnet access.

- Secure all plaintext passwords.

- Provide legal notification.

Configure Router Interfaces

At this point, your routers have their basic configurations. The next step is to configure their interfaces. This is because routers are not reachable by end devices until the interfaces are configured. There are many different types of interfaces available on Cisco routers. For example, the Cisco ISR 4321 router is equipped with two Gigabit Ethernet interfaces:

- GigabitEthernet 0/0/0 (G0/0/0)

- GigabitEthernet 0/0/1 (G0/0/1)

The task to configure a router interface is very similar to a management SVI on a switch. Specifically, it includes issuing the following commands:

Router(config)# interface type-and-number  
  Router(config-if)# description description-text  
  Router(config-if)# ip address  ipv4-address subnet-mask  
  Router(config-if)# ipv6 address  ipv6-address/prefix-length  
  Router(config-if)# no shutdown

Note: When a router interface is enabled, information messages should be displayed confirming the enabled link.

Although the description command is not required to enable an interface, it is good practice to use it. It can be helpful in troubleshooting on production networks by providing information about the type of network connected. For example, if the interface connects to an ISP or service carrier, the description command would be helpful to enter the third-party connection and contact information.

Note: The description-text is limited to 240 characters.

Using the no shutdown command activates the interface and is similar to powering on the interface. The interface must also be connected to another device, such as a switch or a router, for the physical layer to be active.

Note: On inter-router connections where there is no Ethernet switch, both interconnecting interfaces must be configured and enabled.

10.2.2 Configure Router Interfaces Example

In this example, the directly connected interfaces of R1 in the topology diagram will be enabled.

The diagram is a network topology showing the IPv4 and IPv6 addressing of the network devices. What follows is a description of the topology from left to right. PC1 is connected to a switch connected to router R1. The network IPv4 address is 192.168.10.0/24 and the IPv6 address is 2001:db8:acad:10::/64. PC1 has an address of .10 and ::10. Interface G0/0/0 on R1 has an address of .1 and ::1. R1 interface G0/0/1 is then connected to router R2 on IPv4 network 209.165.200.224/30 and IPv6 network 2001:db8:feed:224::/64. Interface G0/0/1 on R1 has an address of .225 and ::1. The interface on R2 has an address of .226 and ::2. R2 is then connected to a switch which is connected to PC2 on IPv4 network 10.1.1.0/24 and IPv6 network 2001:db8:cafe:1::/64. The R2 interface has an address of .1 and ::1. PC2 has an address of .10 and ::10. R2 also has a connection to the Internet cloud.

.10::10192.168.10.0/24.1::1G0/0/0G0/0/1.225::1.226::2.1::1209.165.200.224/3010.1.1.0/24.10::102001:db8:acad:10::/642001:db8:feed:224::/642001:db8:cafe:1::/64PC1PC2R1R2

Internet

To configure the the interfaces on R1, use the following commands.

R1> enable

R1# configure terminal

Enter configuration commands, one per line.

End with CNTL/Z.

R1(config)# interface gigabitEthernet 0/0/0

R1(config-if)# description Link to LAN

R1(config-if)# ip address 192.168.10.1 255.255.255.0

R1(config-if)# ipv6 address 2001:db8:acad:10::1/64

R1(config-if)# no shutdown

R1(config-if)# exit

R1(config)#

*Aug  1 01:43:53.435: %LINK-3-UPDOWN: Interface GigabitEthernet0/0/0, changed state to down

*Aug  1 01:43:56.447: %LINK-3-UPDOWN: Interface GigabitEthernet0/0/0, changed state to up

*Aug  1 01:43:57.447: %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0/0, changed state to up

R1(config)#

R1(config)#

R1(config)# interface gigabitEthernet 0/0/1

R1(config-if)# description Link to R2

R1(config-if)# ip address 209.165.200.225 255.255.255.252

R1(config-if)# ipv6 address 2001:db8:feed:224::1/64

R1(config-if)# no shutdown

R1(config-if)# exit

R1(config)#

*Aug  1 01:46:29.170: %LINK-3-UPDOWN: Interface GigabitEthernet0/0/1, changed state to down

*Aug  1 01:46:32.171: %LINK-3-UPDOWN: Interface GigabitEthernet0/0/1, changed state to up

*Aug  1 01:46:33.171: %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0/1, changed state to up

R1(config)#

Note: Notice the informational messages informing us that G0/0/0 and G0/0/1 are enabled.

10.2.3 Verify Interface Configuration

There are several commands that can be used to verify interface configuration. The most useful of these is the show ip interface brief and show ipv6 interface brief commands, as shown in the example.

R1# show ip interface brief

Interface              IP-Address      OK? Method Status                Protocol

GigabitEthernet0/0/0   192.168.10.1    YES manual up                    up

GigabitEthernet0/0/1   209.165.200.225 YES manual up                    up

Vlan1                  unassigned      YES unset  administratively down down

R1# show ipv6 interface brief

GigabitEthernet0/0/0       [up/up]

FE80::201:C9FF:FE89:4501

2001:DB8:ACAD:10::1

GigabitEthernet0/0/1       [up/up]

FE80::201:C9FF:FE89:4502

2001:DB8:FEED:224::1

Vlan1                      [administratively down/down]

unassigned

R1#

10.2.4 Configuration Verification Commands

The table summarizes the more popular show commands used to verify interface configuration.

| Commands | Description |
| --- | --- |
| show ip interface brief show ipv6 interface brief | The output displays all interfaces, their IP addresses, and their current status. The configured and connected interfaces should display a Status of “up” and Protocol of “up”. Anything else would indicate a problem with either the configuration or the cabling. |
| show ip route show ipv6 route | Displays the contents of the IP routing tables stored in RAM. |
| show interfaces | Displays statistics for all interfaces on the device. However, this command will only display the IPv4 addressing information. |
| show ip interface | Displays the IPv4 statistics for all interfaces on a router. |
| show ipv6 interface | Displays the IPv6 statistics for all interfaces on a router. |

Click each button to see the command output for each configuration verification command.

show ip interface brief

show ipv6 interface brief

show ip route

show ipv6 route

show interfaces

show ip interface

show ipv6 interface

R1# show ip interface brief

Interface              IP-Address      OK? Method Status                Protocol

GigabitEthernet0/0/0   192.168.10.1    YES manual up                    up

GigabitEthernet0/0/1   209.165.200.225 YES manual up                    up

Vlan1                  unassigned      YES unset  administratively down down

R1#

3.3.2  	Default Gateway Configuration

Default Gateway on a Host

If your local network has only one router, it will be the gateway router and all hosts and switches on your network must be configured with this information. If your local network has multiple routers, you must select one of them to be the default gateway router. This topic explains how to configure the default gateway on hosts and switches.

For an end device to communicate over the network, it must be configured with the correct IP address information, including the default gateway address. The default gateway is only used when the host wants to send a packet to a device on another network. The default gateway address is generally the router interface address attached to the local network of the host. The IP address of the host device and the router interface address must be in the same network.

For example, assume an IPv4 network topology consisting of a router interconnecting two separate LANs. G0/0/0 is connected to network 192.168.10.0, while G0/0/1 is connected to network 192.168.11.0. Each host device is configured with the appropriate default gateway address.

In this example, if PC1 sends a packet to PC2, then the default gateway is not used. Instead, PC1 addresses the packet with the IPv4 address of PC2 and forwards the packet directly to PC2 through the switch.

The diagram is a network topology with one router, two switches, and four PCs showing the flow of information between devices on the same network. PC1 and PC2 are connected to the switch on network 192.168.10.0/24 at interface G0/0/0 on router R1. PC3 and PC4 are connected to another switch on network 192.168.11.0/24 at interface G0/0/1 on R1. An arrow shows the flow of information sent from PC1 passing through the attached switch on its way to PC2.

PC!PC!.10.11192.168.10.0/24.1G0/0/0G0/0/1.1.10.11192.168.11.0/24R1PC1PC2PC3PC4

What if PC1 sent a packet to PC3? PC1 would address the packet with the IPv4 address of PC3, but would forward the packet to its default gateway, which is the G0/0/0 interface of R1. The router accepts the packet and accesses its routing table to determine that G0/0/1 is the appropriate exit interface based on the destination address. R1 then forwards the packet out of the appropriate interface to reach PC3.

The diagram is a network topology with one router, two switches, and four PCs showing the flow of information between devices on different networks. PC1 and PC2 are connected to the switch on network 192.168.10.0/24 at interface G0/0/0 on router R1. PC3 and PC4 are connected to another switch on network 192.168.11.0/24 at interface G0/0/1 on R1. An arrow shows the flow of information sent from PC1 passing through R1 and onto PC3.

PC!PC!.10.11192.168.10.0/24.1G0/0/0G0/0/1.1.10.11192.168.11.0/24R1PC1PC2PC3PC4

The same process would occur on an IPv6 network, although this is not shown in the topology. Devices would use the IPv6 address of the local router as their default gateway.

10.3.2 Default Gateway on a Switch

A switch that interconnects client computers is typically a Layer 2 device. As such, a Layer 2 switch does not require an IP address to function properly. However, an IP configuration can be configured on a switch to give an administrator remote access to the switch.

To connect to and manage a switch over a local IP network, it must have a switch virtual interface (SVI) configured. The SVI is configured with an IPv4 address and subnet mask on the local LAN. The switch must also have a default gateway address configured to remotely manage the switch from another network.

The default gateway address is typically configured on all devices that will communicate beyond their local network.

To configure an IPv4 default gateway on a switch, use the ip default-gateway ip-address global configuration command. The ip-address that is configured is the IPv4 address of the local router interface connected to the switch.

The figure shows an administrator establishing a remote connection to switch S1 on another network.

The diagram is a network topology showing a router R1 connected to two switches, S1 on network 192.168.10.0/24, and S2 on network 192.168.11.0/24. A user is connected to S2 and an arrow shows the user is accessing S1 remotely. Above the user is a box showing the user has CLI access to S1 and is displaying the running configuration.

S1# show running-configBuilding configuration...!service password-encryption!hostname S1!Interface Vlan1 ip address 192.168.10.50.255.255.255.0!!ip default-gateway 192.168.10.1.1G0/0/0.1G0/0/1PC1PC2S2.10.11192.168.10.0/24192.168.11.0/24S1R1.50

Output Omitted

Output Omitted

Output Omitted

In this example, the administrator host would use its default gateway to send the packet to the G0/0/1 interface of R1. R1 would forward the packet to S1 out of its G0/0/0 interface. Because the packet source IPv4 address came from another network, S1 would require a default gateway to forward the packet to the G0/0/0 interface of R1. Therefore, S1 must be configured with a default gateway to be able to reply and establish an SSH connection with the administrative host.

Note: Packets originating from host computers connected to the switch must already have the default gateway address configured on their host computer operating systems.

A workgroup switch can also be configured with an IPv6 address on an SVI. However, the switch does not require the IPv6 address of the default gateway to be configured manually. The switch will automatically receive its default gateway from the ICMPv6 Router Advertisement message from the router.