UNIT IV IP ADDRESSING

IPv4 Addressing: Introduction - IPv4 Address Structure - IPv4 Unicast, Broadcast, and

Multicast - Types of IPv4 Addresses -Network Segmentation - Subnet an IPv4

Network - Subnet a Slash 16 and a Slash 8 Prefix - Subnet to Meet Requirements –

VLSM - structured Design.

IPv6 Addressing: Introduction - IPv4 Issues - IPv6 Address Representation - IPv6

Address Types - GUA and LLA Static Configuration - Dynamic Addressing for IPv6

GUAs - Dynamic Addressing for IPv6 LLAs - IPv6 Multicast Addresses - Subnet an

IPv6 Network .

ICMP: Introduction - ICMP Messages - Ping and Traceroute Tests

4.1 IPV4 Addressing

Currently, there are still plenty of networks using IPv4 addressing, even as the organizations which use them are making the transition to IPv6. So it is still very important for network administrators to know everything they can about IPv4 addressing. This module covers the fundamental aspects of IPv4 addressing in detail. It includes how to segment a network into subnets and how to create a variable-length subnet mask (VLSM) as part of an overall IPv4 addressing scheme. Subnetting is like cutting a pie into smaller and smaller pieces. Subnetting may seem overwhelming at first, but we show you some tricks to help you along the way.

4.1.1 IPV4 Address Structure

Network and Host Portions

An IPv4 address is a 32-bit hierarchical address that is made up of a network portion and a host portion. When determining the network portion versus the host portion, you must look at the 32-bit stream, as shown in the figure.

### IPv4 Address

The diagram shows the breakdown of an IPv4 address into the network and host portions. The IPv4 address is 192.168.10.10. Underneath, the address is converted into 11000000 10101000 00001010 00001010. A dashed line shows the separation between the network and host portions. This occurs after the third octet and the 24th bit.

11000000 10101000 0000101000001010 192 . 168 . 10 . 10

The bits within the network portion of the address must be identical for all devices that reside in the same network. The bits within the host portion of the address must be unique to identify a specific host within a network. If two hosts have the same bit-pattern in the specified network portion of the 32-bit stream, those two hosts will reside in the same network.

But how do hosts know which portion of the 32-bits identifies the network and which identifies the host? That is the role of the subnet mask.

The Subnet Mask

As shown in the figure, assigning an IPv4 address to a host requires the following:

- IPv4 address - This is the unique IPv4 address of the host.

- Subnet mask- This is used to identify the network/host portion of the IPv4 address.

### IPv4 Configuration on a Windows Computer

Note: A default gateway IPv4 address is required to reach remote networks and DNS server IPv4 addresses are required to translate domain names to IPv4 addresses.

The IPv4 subnet mask is used to differentiate the network portion from the host portion of an IPv4 address. When an IPv4 address is assigned to a device, the subnet mask is used to determine the network address of the device. The network address represents all the devices on the same network.

The next figure displays the 32-bit subnet mask in dotted decimal and binary formats.

### Subnet Mask

subnet mask of 255.255.255.0 on top with the binary representation of 11111111 111111111 11111111 0000000 underneath; a dashed line is drawn after the third octet and the 24th bit

255 . 255 . 255 . 11111111 000000000 11111111 11111111

Notice how the subnet mask is a consecutive sequence of 1 bits followed by a consecutive sequence of 0 bits.

To identify the network and host portions of an IPv4 address, the subnet mask is compared to the IPv4 address bit for bit, from left to right as shown in the figure.

### Associating an IPv4 Address with its Subnet Mask

The figure shows an IPv4 address, written in both dotted-decimal and binary, with the subnet mask below, also written in dotted-decimal and binary, used to show the division between the network portion and host portion of the address. The IPv4 address is 192.168.10.10 which is converted to 11000000 10101000 00001010 00001010. The subnet mask is 255.255.255.0 which is converted to 11111111 11111111 11111111 00000000. A dashed line shows the separation between the network and host portions. This occurs after the third octet and 24th bit.

255 . 255 . 255 . 0 11000000 10101000 0000101000001010 192 . 168 . 10 . 10 11111111 0000000011111111 11111111

Note that the subnet mask does not actually contain the network or host portion of an IPv4 address, it just tells the computer where to look for the part of the IPv4 address that is the network portion and which part is the host portion.

The actual process used to identify the network portion and host portion is called ANDing.

The Prefix Length

Expressing network addresses and host addresses with the dotted decimal subnet mask address can become cumbersome. Fortunately, there is an alternative method of identifying a subnet mask, a method called the prefix length.

The prefix length is the number of bits set to 1 in the subnet mask. It is written in “slash notation”, which is noted by a forward slash (/) followed by the number of bits set to 1. Therefore, count the number of bits in the subnet mask and prepend it with a slash.

Refer to the table for examples. The first column lists various subnet masks that can be used with a host address. The second column displays the converted 32-bit binary address. The last column displays the resulting prefix length.

### Comparing the Subnet Mask and Prefix Length

| Subnet Mask | 32-bit Address | Prefix Length |
| --- | --- | --- |
| 255.0.0.0 | 11111111.00000000.00000000.00000000 | /8 |
| 255.255.0.0 | 11111111.11111111.00000000.00000000 | /16 |
| 255.255.255.0 | 11111111.11111111.11111111.00000000 | /24 |
| 255.255.255.128 | 11111111.11111111.11111111.10000000 | /25 |
| 255.255.255.192 | 11111111.11111111.11111111.11000000 | /26 |
| 255.255.255.224 | 11111111.11111111.11111111.11100000 | /27 |
| 255.255.255.240 | 11111111.11111111.11111111.11110000 | /28 |
| 255.255.255.248 | 11111111.11111111.11111111.11111000 | /29 |
| 255.255.255.252 | 11111111.11111111.11111111.11111100 | /30 |

Note: A network address is also referred to as a prefix or network prefix. Therefore, the prefix length is the number of 1 bits in the subnet mask.

When representing an IPv4 address using a prefix length, the IPv4 address is written followed by the prefix length with no spaces. For example, 192.168.10.10 255.255.255.0 would be written as 192.168.10.10/24. Using various types of prefix lengths will be discussed later. For now, the focus will be on the /24 (i.e. 255.255.255.0) prefix

Determining the Network - Logical AND

A logical AND is one of three Boolean operations used in Boolean or digital logic. The other two are OR and NOT. The AND operation is used in determining the network address.

Logical AND is the comparison of two bits that produce the results shown below. Note how only a 1 AND 1 produces a 1. Any other combination results in a 0.

- 1 AND 1 = 1

- 0 AND 1 = 0

- 1 AND 0 = 0

- 0 AND 0 = 0

Note: In digital logic, 1 represents True and 0 represents False. When using an AND operation, both input values must be True (1) for the result to be True (1).

To identify the network address of an IPv4 host, the IPv4 address is logically ANDed, bit by bit, with the subnet mask. ANDing between the address and the subnet mask yields the network address.

To illustrate how AND is used to discover a network address, consider a host with IPv4 address 192.168.10.10 and subnet mask of 255.255.255.0, as shown in the figure:

- IPv4 host address (192.168.10.10) - The IPv4 address of the host in dotted decimal and binary formats.

- Subnet mask (255.255.255.0) - The subnet mask of the host in dotted decimal and binary formats.

- Network address (192.168.10.0) - The logical AND operation between the IPv4 address and subnet mask results in an IPv4 network address shown in dotted decimal and binary formats.

The diagram shows the ANDing process between an IPv4 host address and a subnet mask resulting in the IPv4 network address of the host. The IPv4 host address is 192.168.10.10. Below that, the address is converted into 11000000 10101000 00001010 00001010. Below that, the subnet mask of 255.255.255.0 is written. Below that, the subnet mask is converted to 11111111 11111111 11111111 00000000. A line is drawn underneath the binary equivalent of the subnet mask. Below the line is the dotted-decimal and binary equivalent of the IPv4 network address as determined by the ANDing process. A blue shaded box shows the first bit of the IPv4 host address, a 1, compared to the first bit of the subnet mask, also a 1, resulting in a 1 as the first bit value in the IPv4 network address. The IPv4 network address is 192.168.10.0 with a binary equivalent of 11000000 101001000 00001010 00000000.

0000 10101100 00001010 10000000 1010192168...10100000 00001111 11111111 11111111 11110000 00001100 00001010 10000000 1010255255...2550192168...100

IPv4 host
address

Subnet Mask

IPv4
network
address

AND

Equals

Using the first sequence of bits as an example, notice the AND operation is performed on the 1-bit of the host address with the 1-bit of the subnet mask. This results in a 1 bit for the network address. 1 AND 1 = 1.

The AND operation between an IPv4 host address and subnet mask results in the IPv4 network address for this host. In this example, the AND operation between the host address of 192.168.10.10 and the subnet mask 255.255.255.0 (/24), results in the IPv4 network address of 192.168.10.0/24. This is an important IPv4 operation, as it tells the host what network it belongs to.

Network, Host, and Broadcast Addresses

Within each network are three types of IP addresses:

- Network address

- Host addresses

- Broadcast address

Network address

A network address is an address that represents a specific network. A device belongs to this network if it meets three criteria:

- It has the same subnet mask as the network address.

- It has the same network bits as the network address, as indicated by the subnet mask.

- It is located on the same broadcast domain as other hosts with the same network address.

A host determines its network address by performing an AND operation between its IPv4 address and its subnet mask.

As shown in the table, the network address has all 0 bits in the host portion, as determined by the subnet mask. In this example, the network address is 192.168.10.0/24. A network address cannot be assigned to a device.

### Network, Host, and Broadcast Addresses

|  | Network Portion | Host Portion | Host Bits |
| --- | --- | --- | --- |
| Subnet mask 255.255.255.0 or /24 | 255 255 255 11111111 11111111 11111111 | 0 00000000 |  |
| Network address 192.168.10.0 or /24 | 192 168 10 11000000 10101000 00001010 | 0 00000000 | All 0s |
| First address 192.168.10.1 or /24 | 192 168 10 11000000 10101000 00001010 | 1 00000001 | All 0s and a 1 |
| Last address 192.168.10.254 or /24 | 192 168 10 11000000 10101000 00001010 | 254 11111110 | All 1s and a 0 |
| Broadcast address 192.168.10.255 or /24 | 192 168 10 11000000 10101000 00001010 | 255 11111111 | All 1s |

Host addresses

Host addresses are addresses that can be assigned to a device such as a host computer, laptop, smart phone, web camera, printer, router, etc. The host portion of the address is the bits indicated by 0 bits in the subnet mask. Host addresses can have any combination of bits in the host portion except for all 0 bits (this would be a network address) or all 1 bits (this would be a broadcast address).

All devices within the same network, must have the same subnet mask and the same network bits. Only the host bits will differ and must be unique.

Notice that in the table, there is a first and last host address:

- First host address - This first host within a network has all 0 bits with the last (right-most) bit as a 1 bit. In this example it is 192.168.10.1/24.

- Last host address - This last host within a network has all 1 bits with the last (right-most) bit as a 0 bit. In this example it is 192.168.10.254/24.

Any addresses between and including, 192.168.10.1/24 through 192.168.10.254/24 can be assigned to a device on the network.

Broadcast address

A broadcast address is an address that is used when it is required to reach all devices on the IPv4 network. As shown in the table, the network broadcast address has all 1 bits in the host portion, as determined by the subnet mask. In this example, the network address is 192.168.10.255/24. A broadcast address cannot be assigned to a device.

4.1.2 IPV4 Unicast, Multicast and Broad Cast

Unicast

In the previous topic you learned about the structure of an IPv4 address; each has a network portion and a host portion. There are different ways to send a packet from a source device, and these different transmissions affect the destination IPv4 addresses.

Unicast transmission refers to one device sending a message to one other device in one-to-one communications.

A unicast packet has a destination IP address that is a unicast address which goes to a single recipient. A source IP address can only be a unicast address, because the packet can only originate from a single source. This is regardless of whether the destination IP address is a unicast, broadcast or multicast.

IPv4 unicast host addresses are in the address range of 1.0.0.1 to 223.255.255.255. However, within this range are many addresses that are reserved for special purposes. These special purpose addresses will be discussed later in this module.

Broadcast

Broadcast transmission refers to a device sending a message to all the devices on a network in one-to-all communications.

A broadcast packet has a destination IP address with all ones (1s) in the host portion, or 32 one (1) bits.

Note: IPv4 uses broadcast packets. However, there are no broadcast packets with IPv6.

A broadcast packet must be processed by all devices in the same broadcast domain. A broadcast domain identifies all hosts on the same network segment. A broadcast may be directed or limited. A directed broadcast is sent to all hosts on a specific network. For example, a host on the 172.16.4.0/24 network sends a packet to 172.16.4.255. A limited broadcast is sent to 255.255.255.255. By default, routers do not forward broadcasts.

Broadcast packets use resources on the network and make every receiving host on the network process the packet. Therefore, broadcast traffic should be limited so that it does not adversely affect the performance of the network or devices. Because routers separate broadcast domains, subdividing networks can improve network performance by eliminating excessive broadcast traffic.

IP Directed Broadcasts

In addition to the 255.255.255.255 broadcast address, there is a broadcast IPv4 address for each network. Called a directed broadcast, this address uses the highest address in the network, which is the address where all the host bits are 1s. For example, the directed broadcast address for 192.168.1.0/24 is 192.168.1.255. This address allows communication to all the hosts in that network. To send data to all the hosts in a network, a host can send a single packet that is addressed to the broadcast address of the network.

A device that is not directly connected to the destination network forwards an IP directed broadcast in the same way it would forward unicast IP packets destined to a host on that network. When a directed broadcast packet reaches a router that is directly connected to the destination network, that packet is broadcast on the destination network.

Note: Because of security concerns and prior abuse from malicious users, directed broadcasts are turned off by default starting with Cisco IOS Release 12.0 with the global configuration command no ip directed-broadcasts.

Multicast

Multicast transmission reduces traffic by allowing a host to send a single packet to a selected set of hosts that subscribe to a multicast group.

A multicast packet is a packet with a destination IP address that is a multicast address. IPv4 has reserved the 224.0.0.0 to 239.255.255.255 addresses as a multicast range.

Hosts that receive particular multicast packets are called multicast clients. The multicast clients use services requested by a client program to subscribe to the multicast group.

Each multicast group is represented by a single IPv4 multicast destination address. When an IPv4 host subscribes to a multicast group, the host processes packets addressed to this multicast address, and packets addressed to its uniquely allocated unicast address.

Routing protocols such as OSPF use multicast transmissions. For example, routers enabled with OSPF communicate with each other using the reserved OSPF multicast address 224.0.0.5. Only devices enabled with OSPF will process these packets with 224.0.0.5 as the destination IPv4 address. All other devices will ignore these packets.

4.1.3 Types of IPV4 Addresses

Public and Private IPv4 Addresses

Just as there are different ways to transmit an IPv4 packet, there are also different types of IPv4 addresses. Some IPv4 addresses cannot be used to go out to the internet, and others are specifically allocated for routing to the internet. Some are used to verify a connection and others are self-assigned. As a network administrator, you will eventually become very familiar with the types of IPv4 addresses, but for now, you should at least know what they are and when to use them.

Public IPv4 addresses are addresses which are globally routed between internet service provider (ISP) routers. However, not all available IPv4 addresses can be used on the internet. There are blocks of addresses called private addresses that are used by most organizations to assign IPv4 addresses to internal hosts.

In the mid-1990s, with the introduction of the World Wide Web (WWW), private IPv4 addresses were introduced because of the depletion of IPv4 address space. Private IPv4 addresses are not unique and can be used internally within any network.

Note: The long-term solution to IPv4 address depletion was IPv6.

### The Private Address Blocks

| Network Address and Prefix | RFC 1918 Private Address Range |
| --- | --- |
| 10.0.0.0/8 | 10.0.0.0 - 10.255.255.255 |
| 172.16.0.0/12 | 172.16.0.0 - 172.31.255.255 |
| 192.168.0.0/16 | 192.168.0.0 - 192.168.255.255 |

Note: Private addresses are defined in RFC 1918 and sometimes referred to as RFC 1918 address space.

Routing to the Internet

Most internal networks, from large enterprises to home networks, use private IPv4 addresses for addressing all internal devices (intranet) including hosts and routers. However, private addresses are not globally routable.

In the figure, customer networks 1, 2, and 3 are sending packets outside their internal networks. These packets have a source IPv4 address that is a private address and a destination IPv4 address that is public (globally routable). Packets with a private address must be filtered (discarded) or translated to a public address before forwarding the packet to an ISP.

### Private IPv4 Addresses and Network Address Translation (NAT)

The diagram is a network topology with three networks, each connected to a different ISP router. The ISP routers are performing NAT between each network and the Internet.

10.0.0.0/8172.16.0.0/16192.168.0.0/24ISP1ISP2ISP3

This packet has a source IPv4 address that is a private address. I will translate it to a public IPv4 address using NAT

Network 1

Network 2

Network 3

Internet

Before the ISP can forward this packet, it must translate the source IPv4 address, which is a private address, to a public IPv4 address using Network Address Translation (NAT). NAT is used to translate between private IPv4 and public IPv4 addresses. This is usually done on the router that connects the internal network to the ISP network. Private IPv4 addresses in the organization’s intranet will be translated to public IPv4 addresses before routing to the internet.

Note: Although, a device with a private IPv4 address is not directly accessible from another device across the internet, the IETF does not consider private IPv4 addresses or NAT as effective security measures.

Organizations that have resources available to the internet, such as a web server, will also have devices that have public IPv4 addresses. As shown in the figure, this part of the network is known as the DMZ (demilitarized zone). The router in the figure not only performs routing, it also performs NAT and acts as a firewall for security.

The diagram is a network topology showing a router in the center with three connections; one to the company Intranet, one to a DMZ, and one to the Internet. On the left is the Intranet with devices using private IPv4 addresses. At the top, is the DMZ with two servers using public IPv4 addresses. On the right is the Internet cloud. The router is acting as a firewall and performing NAT.

Note: Private IPv4 addresses are commonly used for educational purposes instead of using a public IPv4 address that most likely belongs to an organization.Points

Special Use IPv4 Addresses

There are certain addresses, such as the network address and broadcast address that cannot be assigned to hosts. There are also special addresses that can be assigned to hosts, but with restrictions on how those hosts can interact within the network.

Loopback addresses

Loopback addresses (127.0.0.0 /8 or 127.0.0.1 to 127.255.255.254) are more commonly identified as only 127.0.0.1, these are special addresses used by a host to direct traffic to itself. For example, it can be used on a host to test if the TCP/IP configuration is operational, as shown in the figure. Notice how the 127.0.0.1 loopback address replies to the ping command. Also note how any address within this block will loop back to the local host, which is shown with the second ping in the figure.

### Pinging the Loopback Interface

C:\Users\NetAcad> ping 127.0.0.1

Pinging 127.0.0.1 with 32 bytes of data:

Reply from 127.0.0.1: bytes=32 time<1ms TTL=128

Reply from 127.0.0.1: bytes=32 time<1ms TTL=128

Reply from 127.0.0.1: bytes=32 time<1ms TTL=128

Reply from 127.0.0.1: bytes=32 time<1ms TTL=128

Ping statistics for 127.0.0.1:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss),

Approximate round trip times in milli-seconds:

Minimum = 0ms, Maximum = 0ms, Average = 0ms

C:\Users\NetAcad> ping 127.1.1.1

Pinging 127.1.1.1 with 32 bytes of data:

Reply from 127.1.1.1: bytes=32 time<1ms TTL=128

Reply from 127.1.1.1: bytes=32 time<1ms TTL=128

Reply from 127.1.1.1: bytes=32 time<1ms TTL=128

Reply from 127.1.1.1: bytes=32 time<1ms TTL=128

Ping statistics for 127.1.1.1:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss),

Approximate round trip times in milli-seconds:

Minimum = 0ms, Maximum = 0ms, Average = 0ms

C:\Users\NetAcad>

Link-Local addresses

Link-local addresses (169.254.0.0 /16 or 169.254.0.1 to 169.254.255.254) are more commonly known as the Automatic Private IP Addressing (APIPA) addresses or self-assigned addresses. They are used by a Windows DHCP client to self-configure in the event that there are no DHCP servers available. Link-local addresses can be used in a peer-to-peer connection but are not commonly used for this purpose.

Legacy Classful Addressing

In 1981, IPv4 addresses were assigned using classful addressing as defined in RFC 790 (https://tools.ietf.org/html/rfc790), Assigned Numbers. Customers were allocated a network address based on one of three classes, A, B, or C. The RFC divided the unicast ranges into specific classes as follows:

- Class A (0.0.0.0/8 to 127.0.0.0/8) - Designed to support extremely large networks with more than 16 million host addresses. Class A used a fixed /8 prefix with the first octet to indicate the network address and the remaining three octets for host addresses (more than 16 million host addresses per network).

- Class B (128.0.0.0 /16 - 191.255.0.0 /16) - Designed to support the needs of moderate to large size networks with up to approximately 65,000 host addresses. Class B used a fixed /16 prefix with the two high-order octets to indicate the network address and the remaining two octets for host addresses (more than 65,000 host addresses per network).

- Class C (192.0.0.0 /24 - 223.255.255.0 /24) - Designed to support small networks with a maximum of 254 hosts. Class C used a fixed /24 prefix with the first three octets to indicate the network and the remaining octet for the host addresses (only 254 host addresses per network).

Note: There is also a Class D multicast block consisting of 224.0.0.0 to 239.0.0.0 and a Class E experimental address block consisting of 240.0.0.0 - 255.0.0.0.

At the time, with a limited number of computers using the internet, classful addressing was an effective means to allocate addresses. As shown in the figure, Class A and B networks have a very large number of host addresses and Class C has very few. Class A networks accounted for 50% of the IPv4 networks. This caused most of the available IPv4 addresses to go unused.

The diagram is a pie chart showing the percentage of Class A, B, C, D, & E IPv4 addressing with the total number of networks and hosts per class A, B, and C networks. Percentages are: class A = 50%, class B = 25%, class C = 12.5%, and class D and E = 12.5%. For the total number of networks and total number of hosts per network: class A = 128 networks with 16,777,214 total hosts per network; class B = 16,384 networks with 65,534 total hosts per network; and class C = 2,097,152 networks with 254 total hosts per network.

4.1.4 Network Segmentation Class D & E
12.5%

1Broadcast Domains and Segmentation

Have you ever received an email that was addressed to every person at your work or school? This was a broadcast email. Hopefully, it contained information that each of you needed to know. But often a broadcast is not really pertinent to everyone in the mailing list. Sometimes, only a segment of the population needs to read that information.

In an Ethernet LAN, devices use broadcasts and the Address Resolution Protocol (ARP) to locate other devices.. ARP sends Layer 2 broadcasts to a known IPv4 address on the local network to discover the associated MAC address. Devices on Ethernet LANs also locate other devices using services. A host typically acquires its IPv4 address configuration using the Dynamic Host Configuration Protocol (DHCP) which sends broadcasts on the local network to locate a DHCP server.

Switches propagate broadcasts out all interfaces except the interface on which it was received. For example, if a switch in the figure were to receive a broadcast, it would forward it to the other switches and other users connected in the network.

### Routers Segment Broadcast Domains

A router, R1, is connected to a switch via interface G0/0. The switch has connections to three other switches. The broadcast domain consists of the four switches and the router interface to which they are connected. A connection from the router to the Internet is not within the broadcast domain.

R1G0/0

Internet

Broadcast Domain

Routers do not propagate broadcasts. When a router receives a broadcast, it does not forward it out other interfaces. For instance, when R1 receives a broadcast on its Gigabit Ethernet 0/0 interface, it does not forward out another interface.

Therefore, each router interface connects to a broadcast domain and broadcasts are only propagated within that specific broadcast domain.

Problems with Large Broadcast Domains

A large broadcast domain is a network that connects many hosts. A problem with a large broadcast domain is that these hosts can generate excessive broadcasts and negatively affect the network. In the figure, LAN 1 connects 400 users that could generate an excess amount of broadcast traffic. This results in slow network operations due to the significant amount of traffic it can cause, and slow device operations because a device must accept and process each broadcast packet.

### A Large Broadcast Domain

A router, R1, is connected to a switch via interface G0/0. The switch has connections to three other switches. The broadcast domain consists of the four switches and the router interface to which they are connected. This is identified as LAN1 with an address of 172.16.0.0/16. A connection from the router to the Internet is not within the broadcast domain.

R1G0/0LAN 1: 172.16.0.0/16

The solution is to reduce the size of the network to create smaller broadcast domains in a process called subnetting. These smaller network spaces are called subnets.

In the figure, the 400 users in LAN 1 with network address 172.16.0.0 /16 have been divided into two subnets of 200 users each: 172.16.0.0 /24 and 172.16.1.0 /24. Broadcasts are only propagated within the smaller broadcast domains. Therefore, a broadcast in LAN 1 would not propagate to LAN 2.

### Communicating Between Networks

A router, R1, is connected to two LANs which represent two different broadcast domains. Connected on the left via G0/0 is a switch supporting 200 users in LAN 1 with a network address of 172.16.0.0/24. Connected on the right via G0/1 is a switch supporting 200 users in LAN 2 with a network address of 172.16.1.0/24.

LAN 1: 172.16.0.0/24LAN 2: 172.16.1.0/24G0/1R1G0/0

Internet

(200 users)

(200 users)

Notice how the prefix length has changed from a single /16 network to two /24 networks. This is the basis of subnetting: using host bits to create additional subnets.

Note: The terms subnet and network are often used interchangeably. Most networks are a subnet of some larger address block.

Reasons for Segmenting Networks

Subnetting reduces overall network traffic and improves network performance. It also enables an administrator to implement security policies such as which subnets are allowed or not allowed to communicate together. Another reason is that it reduces the number of devices affected by abnormal broadcast traffic due to misconfigurations, hardware/software problems, or malicious intent.

There are various ways of using subnets to help manage network devices.

Click each image for an illustration of how network administrators can group devices and services into subnets.

Location

Group or Function

Device Type

Subnetting by Location

The diagram shows a five floor building with a switch on each floor. Each switch is on a different LAN/subnet with a different network address, all connected to the same router, R1, via a different gigabit Ethernet interface. The following subnets are shown from the first to the fifth floor: LAN 1 has a network address of 10.0.1.0/24 and is connected to G0/0; LAN 2 has a network address of 10.0.2.0/24 and is connected to G0/1; LAN 3 has a network address of 10.0.3.0/24 and is connected to G0/2; LAN 4 has a network address of 10.0.4.0/24 and is connected to G0/3; and LAN 5 has a network address of 10.0.5.0/24 and is connected to G0/4. R1 also has a connection to the Internet.

R1G0/0G0/1G0/2G0/3G0/4

LAN 5: 10.0.5.0 /24 (Fifth floor)

LAN 4: 10.0.4.0 /24 (Fourth floor)

LAN 3: 10.0.3.0 /24 (Third floor)

LAN 2: 10.0.2.0 /24 (Second floor)

LAN 1: 10.0.1.0 /24(First floor)

Internet

Network administrators can create subnets using any other division that makes sense for the network. Notice in each figure, the subnets use longer prefix lengths to identify networks.

Understanding how to subnet networks is a fundamental skill that all network administrators must develop. Various methods have been created to help understand this process. Although a little overwhelming at first, pay close attention to the detail and, with practice, subnetting will become easier.

4.1.5 Subnet an IPV4 Network

Subnet on an Octet Boundary

In the previous topic you learned several good reasons for segmenting a network. You also learned that segmenting a network is called subnetting. Subnetting is a critical skill to have when administering an IPv4 network. It is a bit daunting at first, but it gets much easier with practice.

IPv4 subnets are created by using one or more of the host bits as network bits. This is done by extending the subnet mask to borrow some of the bits from the host portion of the address to create additional network bits. The more host bits that are borrowed, the more subnets that can be defined. The more bits that are borrowed to increase the number of subnets reduces the number of hosts per subnet.

Networks are most easily subnetted at the octet boundary of /8, /16, and /24. The table identifies these prefix lengths. Notice that using longer prefix lengths decreases the number of hosts per subnet.

### Subnet Masks on Octet Boundaries

| Prefix Length | Subnet Mask | Subnet Mask in Binary (n = network, h = host) | # of hosts |
| --- | --- | --- | --- |
| /8 | 255.0.0.0 | nnnnnnnn.hhhhhhhh.hhhhhhhh.hhhhhhhh 11111111.00000000.00000000.00000000 | 16,777,214 |
| /16 | 255.255.0.0 | nnnnnnnn.nnnnnnnn.hhhhhhhh.hhhhhhhh 11111111.11111111.00000000.00000000 | 65,534 |
| /24 | 255.255.255.0 | nnnnnnnn.nnnnnnnn.nnnnnnnn.hhhhhhhh 11111111.11111111.11111111.00000000 | 254 |

To understand how subnetting on the octet boundary can be useful, consider the following example. Assume an enterprise has chosen the private address 10.0.0.0/8 as its internal network address. That network address can connect 16,777,214 hosts in one broadcast domain. Obviously, having more than 16 million hosts on a single subnet is not ideal.

The enterprise could further subnet the 10.0.0.0/8 address at the octet boundary of /16 as shown in the table. This would provide the enterprise the ability to define up to 256 subnets (i.e., 10.0.0.0/16 - 10.255.0.0/16) with each subnet capable of connecting 65,534 hosts. Notice how the first two octets identify the network portion of the address whereas the last two octets are for host IP addresses.

### Subnetting Network 10.0.0.0/8 using a /16

| Subnet Address (256 Possible Subnets) | Host Range (65,534 possible hosts per subnet) | Broadcast |
| --- | --- | --- |
| 10.0.0.0/16 | 10.0.0.1 - 10.0.255.254 | 10.0.255.255 |
| 10.1.0.0/16 | 10.1.0.1 - 10.1.255.254 | 10.1.255.255 |
| 10.2.0.0/16 | 10.2.0.1 - 10.2.255.254 | 10.2.255.255 |
| 10.3.0.0/16 | 10.3.0.1 - 10.3.255.254 | 10.3.255.255 |
| 10.4.0.0/16 | 10.4.0.1 - 10.4.255.254 | 10.4.255.255 |
| 10.5.0.0/16 | 10.5.0.1 - 10.5.255.254 | 10.5.255.255 |
| 10.6.0.0/16 | 10.6.0.1 - 10.6.255.254 | 10.6.255.255 |
| 10.7.0.0/16 | 10.7.0.1 - 10.7.255.254 | 10.7.255.255 |
| ... | ... | ... |
| 10.255.0.0/16 | 10.255.0.1 - 10.255.255.254 | 10.255.255.255 |

Alternatively, the enterprise could choose to subnet the 10.0.0.0/8 network at the /24 octet boundary, as shown in the table. This would enable the enterprise to define 65,536 subnets each capable of connecting 254 hosts. The /24 boundary is very popular in subnetting because it accommodates a reasonable number of hosts and conveniently subnets at the octet boundary.

### Subnetting Network 10.0.0.0/8 using a /24 Prefix

| Subnet Address (65,536 Possible Subnets) | Host Range (254 possible hosts per subnet) | Broadcast |
| --- | --- | --- |
| 10.0.0.0/24 | 10.0.0.1 - 10.0.0.254 | 10.0.0.255 |
| 10.0.1.0/24 | 10.0.1.1 - 10.0.1.254 | 10.0.1.255 |
| 10.0.2.0/24 | 10.0.2.1 - 10.0.2.254 | 10.0.2.255 |
| … | … | … |
| 10.0.255.0/24 | 10.0.255.1 - 10.0.255.254 | 10.0.255.255 |
| 10.1.0.0/24 | 10.1.0.1 - 10.1.0.254 | 10.1.0.255 |
| 10.1.1.0/24 | 10.1.1.1 - 10.1.1.254 | 10.1.1.255 |
| 10.1.2.0/24 | 10.1.2.1 - 10.1.2.254 | 10.1.2.255 |
| … | … | … |
| 10.100.0.0/24 | 10.100.0.1 - 10.100.0.254 | 10.100.0.255 |
| ... | ... | ... |
| 10.255.255.0/24 | 10.255.255.1 - 10.2255.255.254 | 10.255.255.255 |

11.5.2 Subnet within an Octet Boundary

The examples shown thus far borrowed host bits from the common /8, /16, and /24 network prefixes. However, subnets can borrow bits from any host bit position to create other masks.

For instance, a /24 network address is commonly subnetted using longer prefix lengths by borrowing bits from the fourth octet. This provides the administrator with additional flexibility when assigning network addresses to a smaller number of end devices.

Refer to the table to see six ways to subnet a /24 network.

### Subnet a /24 Network

| Prefix Length | Subnet Mask | Subnet Mask in Binary (n = network, h = host) | # of subnets | # of hosts |
| --- | --- | --- | --- | --- |
| /25 | 255.255.255.128 | nnnnnnnn.nnnnnnnn.nnnnnnnn.nhhhhhhh 11111111.11111111.11111111.10000000 | 2 | 126 |
| /26 | 255.255.255.192 | nnnnnnnn.nnnnnnnn.nnnnnnnn.nnhhhhhh 11111111.11111111.11111111.11000000 | 4 | 62 |
| /27 | 255.255.255.224 | nnnnnnnn.nnnnnnnn.nnnnnnnn.nnnhhhhh 11111111.11111111.11111111.11100000 | 8 | 30 |
| /28 | 255.255.255.240 | nnnnnnnn.nnnnnnnn.nnnnnnnn.nnnnhhhh 11111111.11111111.11111111.11110000 | 16 | 14 |
| /29 | 255.255.255.248 | nnnnnnnn.nnnnnnnn.nnnnnnnn.nnnnnhhh 11111111.11111111.11111111.11111000 | 32 | 6 |
| /30 | 255.255.255.252 | nnnnnnnn.nnnnnnnn.nnnnnnnn.nnnnnnhh 11111111.11111111.11111111.11111100 | 64 | 2 |

For each bit borrowed in the fourth octet, the number of subnetworks available is doubled, while reducing the number of host addresses per subnet:

- /25 row - Borrowing 1 bit from the fourth octet creates 2 subnets supporting 126 hosts each.

- /26 row - Borrowing 2 bits creates 4 subnets supporting 62 hosts each.

- /27 row - Borrowing 3 bits creates 8 subnets supporting 30 hosts each.

- /28 row - Borrowing 4 bits creates 16 subnets supporting 14 hosts each.

- /29 row - Borrowing 5 bits creates 32 subnets supporting 6 hosts each.

- /30 row - Borrowing 6 bits creates 64 subnets supporting 2 hosts each.

4.1.6 Subnet a Slash 16 and a Slash 8 prefix

Create Subnets with a Slash 16 prefix

Some subnetting is easier than other subnetting. This topic explains how to create subnets that each have the same number of hosts.

In a situation requiring a larger number of subnets, an IPv4 network is required that has more hosts bits available to borrow. For example, the network address 172.16.0.0 has a default mask of 255.255.0.0, or /16. This address has 16 bits in the network portion and 16 bits in the host portion. The 16 bits in the host portion are available to borrow for creating subnets. The table highlights all the possible scenarios for subnetting a /16 prefix.

### Subnet a /16 Network

| Prefix Length | Subnet Mask | Network Address (n = network, h = host) | # of subnets | # of hosts |
| --- | --- | --- | --- | --- |
| /17 | 255.255.128.0 | nnnnnnnn.nnnnnnnn.nhhhhhhh.hhhhhhhh 11111111.11111111.10000000.00000000 | 2 | 32766 |
| /18 | 255.255.192.0 | nnnnnnnn.nnnnnnnn.nnhhhhhh.hhhhhhhh 11111111.11111111.11000000.00000000 | 4 | 16382 |
| /19 | 255.255.224.0 | nnnnnnnn.nnnnnnnn.nnnhhhhh.hhhhhhhh 11111111.11111111.11100000.00000000 | 8 | 8190 |
| /20 | 255.255.240.0 | nnnnnnnn.nnnnnnnn.nnnnhhhh.hhhhhhhh 11111111.11111111.11110000.00000000 | 16 | 4094 |
| /21 | 255.255.248.0 | nnnnnnnn.nnnnnnnn.nnnnnhhh.hhhhhhhh 11111111.11111111.11111000.00000000 | 32 | 2046 |
| /22 | 255.255.252.0 | nnnnnnnn.nnnnnnnn.nnnnnnhh.hhhhhhhh 11111111.11111111.11111100.00000000 | 64 | 1022 |
| /23 | 255.255.254.0 | nnnnnnnn.nnnnnnnn.nnnnnnnh.hhhhhhhh 11111111.11111111.11111110.00000000 | 128 | 510 |
| /24 | 255.255.255.0 | nnnnnnnn.nnnnnnnn.nnnnnnnn.hhhhhhhh 11111111.11111111.11111111.00000000 | 256 | 254 |
| /25 | 255.255.255.128 | nnnnnnnn.nnnnnnnn.nnnnnnnn.nhhhhhhh 11111111.11111111.11111111.10000000 | 512 | 126 |
| /26 | 255.255.255.192 | nnnnnnnn.nnnnnnnn.nnnnnnnn.nnhhhhhh 11111111.11111111.11111111.11000000 | 1024 | 62 |
| /27 | 255.255.255.224 | nnnnnnnn.nnnnnnnn.nnnnnnnn.nnnhhhhh 11111111.11111111.11111111.11100000 | 2048 | 30 |
| /28 | 255.255.255.240 | nnnnnnnn.nnnnnnnn.nnnnnnnn.nnnnhhhh 11111111.11111111.11111111.11110000 | 4096 | 14 |
| /29 | 255.255.255.248 | nnnnnnnn.nnnnnnnn.nnnnnnnn.nnnnnhhh 11111111.11111111.11111111.11111000 | 8192 | 6 |
| /30 | 255.255.255.252 | nnnnnnnn.nnnnnnnn.nnnnnnnn.nnnnnnhh 11111111.11111111.11111111.11111100 | 16384 | 2 |

Although you do not need to memorize this table, you still need a good understanding of how each value in the table is generated. Do not let the size of the table intimidate you. The reason it is big is that it has 8 additional bits that can be borrowed, and, therefore, the numbers of subnets and hosts are simply larger.

Create 100 Subnets with a Slash 16 prefix

Consider a large enterprise that requires at least 100 subnets and has chosen the private address 172.16.0.0/16 as its internal network address.

When borrowing bits from a /16 address, start borrowing bits in the third octet, going from left to right. Borrow a single bit at a time until the number of bits necessary to create 100 subnets is reached.

The figure displays the number of subnets that can be created when borrowing bits from the third octet and the fourth octet. Notice there are now up to 14 host bits that can be borrowed.

### Number of Subnets Created

The graphic shows how to compute the number of subnets created when borrowing bits from the third and fourth octets of an IPv4 network address. The formula to determine the number of subnets created is 2 to the power of the number of bits borrowed. The graphic shows an address of 172.16.0.0. Underneath, are the letters nnnnnnnn.nnnnnnnn.hhhhhhhh.hhhhhhhh. It starts by borrowing the first h bit in the third octet which results in 2 to the power of 1 = 2 subnets. When the first two h bits in the third octet are borrowed, the formula is 2 to the power of 2 = 4. This continues until the first 14 h bits are borrowed from the third and fourth octets resulting in 2 to the power of 14 = 16384. The last two h bits in the fourth octet remain the same.

nnnnnnnn.nnnnnnnn.hhhhhhhh.hhhhhhhh172.016.0.

To satisfy the requirement of 100 subnets for the enterprise, 7 bits (i.e., 27 = 128 subnets) would need to be borrowed (for a total of 128 subnets), as shown in the figure.

### 172.16.0.0/23 Network

The graphic shows the decimal and bit representation of a network address, and below it a subnet mask, when seven bits are borrowed in the third octet to create subnets. The first two octets are shown in decimal and the last two octets are shown in binary. The network address is 172.16.0000 0000.0000 0000. The subnet mask is 255.255.1111 1110.0000 0000.

172. 16. 0000 000 0. 0000 0000255. 255. 1111 111 0. 0000 0000

Recall that the subnet mask must change to reflect the borrowed bits. In this example, when 7 bits are borrowed, the mask is extended 7 bits into the third octet. In decimal, the mask is represented as 255.255.254.0, or a /23 prefix, because the third octet is 11111110 in binary and the fourth octet is 00000000 in binary.

The figure displays the resulting subnets from 172.16.0.0 /23 up to 172.16.254.0 /23.

### Resulting /23 Subnets

The graphic shows the subnets created when using a /23 subnet mask with the address 172.16.0.0. First, it shows the decimal and bit representation of the network address, and below it the subnet mask. The first two octets are shown in decimal and the last two octets are shown in binary. The network address is 172.16.0000 0000.0000 0000. The subnet mask is 255.255.1111 1110.0000 0000. The first two octets and the first seven bits in the third octet are shaded gray and the last bit in the third octet and the entire fourth octet are shaded purple. Below, the text reads: borrowing 7 bits creates 128 subnets. Below that, it shows the first three subnets and the last subnet created. Again, the first two octets are shown in decimal and the last two octets are shown in binary. The first subnet is 172.16.0000 0000.0000 0000 or 172.16.0.0/23. The second subnet is 172.16.0000 0010.0000 0000 or 172.16.2.0/23. The third subnet is 172.16.0000 0100.0000 0000 or 172.16.4.0/23. The text ..to.. is used to show that this process continues until you reach the last subnet created which is 172.16.1111 1110.0000 0000 or 172.16.254.0/23.

172.16.0.0/23172.16.2.0/23172.16.4.0/23172.16.00000000.00000000255.255.11111110.00000000172.16.254.0/23172.16.00000000.00000000172.16.00000010.00000000172.16.00000100.00000000172.16.11111110.00000000

Borrowing 7 bits creates 128 subnets

. . to . .

After borrowing 7 bits for the subnet, there is one host bit remaining in the third octet, and 8 host bits remaining in the fourth octet, for a total of 9 bits that were not borrowed. 29 results in 512 total host addresses. The first address is reserved for the network address and the last address is reserved for the broadcast address, so subtracting for these two addresses (29 - 2) equals 510 available host addresses for each /23 subnet.

As shown in the figure, the first host address for the first subnet is 172.16.0.1, and the last host address is 172.16.1.254.

### Address Range for 172.16.0.0/23 Subnet

The graphic shows the address range for the 172.16.0.0/23 subnet. The first two octets are shown in decimal and the last two octets are shown in binary, then the address is shown in its dotted decimal format. The network address is 172.16.0000 0000.0000 0000 = 172.16.0.0/23. The first host address is 172.16.0000 0000.0000 0001 = 172.16.0.1/23. The last host address is 172.16.0000 0001.1111 1110 = 172.16.255.254/23 (change to 172.16.1.254 when fixed). The broadcast address is 172.16.0000 0001.1111 1111 = 172.16.255.255/23 (change to 172.16.1.255 when fixed).

172.16.00000000.00000000172.16.00000000.00000001172.16.00000001.11111110172.16.00000001.11111111= 172.16.0.0/23= 172.16.0.1/23= 172.16.1.254/23= 172.16.1.255/23

Network Address

First Host Address

Last Host Address

Broadcast Address

Create 1000 Subnets with a Slash 8 prefix

Some organizations, such as small service providers or large enterprises, may need even more subnets. For example, take a small ISP that requires 1000 subnets for its clients. Each client will need plenty of space in the host portion to create its own subnets.

The ISP has a network address 10.0.0.0 255.0.0.0 or 10.0.0.0/8. This means there are 8 bits in the network portion and 24 host bits available to borrow toward subnetting. Therefore, the small ISP will subnet the 10.0.0.0/8 network.

To create subnets, you must borrow bits from the host portion of the IPv4 address of the existing internetwork. Starting from the left to the right with the first available host bit, borrow a single bit at a time until you reach the number of bits necessary to create 1000 subnets. As shown in the figure, you need to borrow 10 bits to create 1024 subnets (210 = 1024). This includes 8 bits in the second octet and 2 additional bits from the third octet.

### Number of Subnets Created

The graphic shows how to compute the number of subnets created when borrowing bits from the second and third octets of an IPv4 network address. The formula to determine the number of subnets created is 2 to the power of the number of bits borrowed. The graphic shows an address of 10.0.0.0. Underneath, are the letters nnnnnnnn.hhhhhhhh.hhhhhhhh.hhhhhhhh. It starts by borrowing the first h bit in the second octet which results in 2 to the power of 1 = 2 subnets. When the first two h bits in the second octet are borrowed, the formula is 2 to the power of 2 = 4. This continues until the first 10 h bits are borrowed from the second and third octets resulting in 2 to the power of 10 = 1024.

nnnnnnnn.hhhhhhhh.hhhhhhhh.hhhhhhhh10.00.0.2^1 = 22^2 = 42^3 = 82^4 = 162^5 = 322^6 = 642^7 = 1282^8 = 2562^9 = 5122^10 = 1024

### 10.0.0.0/18 Network

The graphic shows the decimal and bit representation of a network address, and below it a subnet mask, when 10 bits are borrowed in the second and third octets to create subnets. The first octet is shown in decimal and the last three octets are shown in binary. The network address is 10.1111 1111.1100 0000.0000 0000 (should be 10.0000 0000.0000 0000.0000 0000 when fixed). The subnet mask is 255.255.1111 1110.0000 0000.

10.00000000.00000000.00000000255.11111111.11000000.00000000

This figure displays the subnets resulting from borrowing 10 bits, creating subnets from 10.0.0.0/18 to 10.255.192.0/18.

### Resulting /18 Subnets

The graphic shows the subnets created when using a /18 subnet mask with the address 10.0.0.0. First, it shows the decimal and bit representation of the network address, and below it the subnet mask. The first octet is shown in decimal and the last three octets are shown in binary. The network address is 10.0000 0000.0000 0000.0000 0000. The subnet mask is 255.1111 1111.1100 0000.0000 0000. The first octet and the next 10 bits are shaded gray and the remaining bits are shaded purple. Below, the text reads: borrowing 10 bits creates 1024 subnets. Below that, it shows the first five subnets and the last subnet created. Again, the first octet is shown in decimal and the last three octets are shown in binary. The first subnet is 10.0000 0000.0000 0000.0000 0000 or 10.0.0.0/18. The second subnet is 10.0000 0000.0100 0000.0000 0000 or 10.0.64.0/18. The third subnet is 10.0000 0000.1000 0000.0000 0000 or 10.0.128.0/18. The fourth subnet is 10.0000 0000.1100 0000.0000 0000 or 10.0.192.0/18. The fifth subnet is 10.0000 0001.0000 0000.0000 0000 or 10.1.0.0/18. The text ..to.. is used to show that this process continues until you reach the last subnet created which is 10.1111 1111.1100 0000.0000 0000 or 10.255.192.0/18.

10.0.0.0/1810.0.64.0/1810.0.128.0/18255.11111111.11000000.0000000010.00000000.00000000.0000000010.00000000.00000000.0000000010.00000000.01000000.0000000010.00000000.10000000.0000000010.0.192.0/1810.1.0.0/1810.00000000.11000000.0000000010.00000001.00000000.0000000010.11111111.11000000.0000000010.255.192.0/18

Borrowing 10 bits creates 1024 subnets

. . to . .

Borrowing 10 bits to create the subnets, leaves 14 host bits for each subnet. Subtracting two hosts per subnet (one for the network address and one for the broadcast address) equates to 214 - 2 = 16382 hosts per subnet. This means that each of the 1000 subnets can support up to 16,382 hosts.

This figure displays the specifics of the first subnet.

### Address Range for 10.0.0.0/18 Subnet

The graphic shows the address range for the 10.0.0.0/18 subnet. The first octet is shown in decimal and the last three octets are shown in binary, then the address is shown in its dotted decimal format. The network address is 10.0000 0000.0000 0000.0000 0000 = 10.0.0.0/18. The first host address is 10.0000 0000.0000 0000.0000 0001 = 10.0.0.1/18. The last host address is 10.0000 0000.0011 1111.1111 1110 = 10.0.63.254/18. The broadcast address is 10.0000 0000.0011 1111. 1111 1111 = 10.0.63.255/18.

10.00000000.00000000.0000000110.00000000.00000000.0000000010.00000000.00111111.1111111110.00000000.00111111.11111110= 10.0.0.0/18= 10.0.0.1/18= 10.0.63.254/18= 10.0.63.255/18

4.1.7 Subnet to Meet Requirements

Subnet Private versus Public IPv4 Address Space

While it is nice to quickly segment a network into subnets, your organization’s network may use both public and private IPv4 addresses. This affects how you will subnet your network.

The figure shows a typical enterprise network:

- Intranet - This is the internal part of a company’s network, accessible only within the organization. Devices in the intranet use private IPv4 addresses.

- DMZ - This is part of the company’s network containing resources available to the internet such as a web server. Devices in the DMZ use public IPv4 addresses.

### Public and Private IPv4 Address Space

The diagram is a network topology showing a router in the center with three connections; one to the company Intranet, one to a DMZ, and one to the Internet. On the left is the Intranet with devices using private IPv4 addresses. At the top, is the DMZ with two servers using public IPv4 addresses. The router is labeled router to the Internet and has a connection to the Internet cloud.

Internet

Router to the Internet

Private IPv4 Addresses

Intranet

Public IPv4 Addresses

DMZ

Both the intranet and the DMZ have their own subnetting requirements and challenges.

The intranet uses private IPv4 addressing space. This allows an organization to use any of the private IPv4 network addresses including the 10.0.0.0/8 prefix with 24 host bits and over 16 million hosts. Using a network address with 24 host bits makes subnetting easier and more flexible. This includes subnetting on an octet boundary using a /16 or /24.

For example, the private IPv4 network address 10.0.0.0/8 can be subnetted using a /16 mask. As shown in the table, this results in 256 subnets, with 65,534 hosts per subnet. If an organization has a need for fewer than 200 subnets, allowing for some growth, this gives each subnet more than enough host addresses.

### Subnetting Network 10.0.0.0/8 using a /16

| Subnet Address (256 Possible Subnets) | Host Range (65,534 possible hosts per subnet) | Broadcast |
| --- | --- | --- |
| 10.0.0.0/16 | 10.0.0.1 - 10.0.255.254 | 10.0.255.255 |
| 10.1.0.0/16 | 10.1.0.1 - 10.1.255.254 | 10.1.255.255 |
| 10.2.0.0/16 | 10.2.0.1 - 10.2.255.254 | 10.2.255.255 |
| 10.3.0.0/16 | 10.3.0.1 - 10.3.255.254 | 10.3.255.255 |
| 10.4.0.0/16 | 10.4.0.1 - 10.4.255.254 | 10.4.255.255 |
| 10.5.0.0/16 | 10.5.0.1 - 10.5.255.254 | 10.5.255.255 |
| 10.6.0.0/16 | 10.6.0.1 - 10.6.255.254 | 10.6.255.255 |
| 10.7.0.0/16 | 10.7.0.1 - 10.7.255.254 | 10.7.255.255 |
| ... | ... | ... |
| 10.255.0.0/16 | 10.255.0.1 - 10.255.255.254 | 10.255.255.255 |

Another option using the 10.0.0.0/8 private IPv4 network address is to subnet using a /24 mask. As shown in the table, this results in 65,536 subnets, with 254 hosts per subnet. If an organization needs more than 256 subnets, then using a /24 can be used with 254 hosts per subnet.

### Subnetting Network 10.0.0.0/8 using a /24

| Subnet Address (65,536 Possible Subnets) | Host Range (254 possible hosts per subnet) | Broadcast |
| --- | --- | --- |
| 10.0.0.0/24 | 10.0.0.1 - 10.0.0.254 | 10.0.0.255 |
| 10.0.1.0/24 | 10.0.1.1 - 10.0.1.254 | 10.0.1.255 |
| 10.0.2.0/24 | 10.0.2.1 - 10.0.2.254 | 10.0.2.255 |
| … | … | … |
| 10.0.255.0/24 | 10.0.255.1 - 10.0.255.254 | 10.0.255.255 |
| 10.1.0.0/24 | 10.1.0.1 - 10.1.0.254 | 10.1.0.255 |
| 10.1.1.0/24 | 10.1.1.1 - 10.1.1.254 | 10.1.1.255 |
| 10.1.2.0/24 | 10.1.2.1 - 10.1.2.254 | 10.1.2.255 |
| … | … | … |
| 10.100.0.0/24 | 10.100.0.1 - 10.100.0.254 | 10.100.0.255 |
| ... | ... | ... |
| 10.255.255.0/24 | 10.255.255.1 - 10.2255.255.254 | 10.255.255.255 |

The 10.0.0.0/8 can also be subnetted using any other number of prefix lengths, such as /12, /18, /20, etc. This would give the network administrator a wide variety of options. Using a 10.0.0.0/8 private IPv4 network address makes subnet planning and implementation easy.

What about the DMZ?

Because these devices need to be publicly accessible from the internet, the devices in the DMZ require public IPv4 addresses. The depletion of public IPv4 address space became an issue beginning in the mid-1990s. Since 2011, IANA and four out of five RIRs have run out of IPv4 address space. Although organizations are making the transition to IPv6, the remaining IPv4 address space remains severely limited. This means an organization must maximize its own limited number of public IPv4 addresses. This requires the network administrator to subnet their public address space into subnets with different subnet masks, in order to minimize the number of unused host addresses per subnet. This is known as Variable Subnet Length Masking (VLSM).

Minimize Unused Host IPv4 Addresses and Maximize Subnets

To minimize the number of unused host IPv4 addresses and maximize the number of available subnets, there are two considerations when planning subnets: the number of host addresses required for each network and the number of individual subnets needed.

The table displays the specifics for subnetting a /24 network. Notice how there is an inverse relationship between the number of subnets and the number of hosts. The more bits that are borrowed to create subnets, the fewer host bits remain available. If more host addresses are needed, more host bits are required, resulting in fewer subnets.

The number of host addresses required in the largest subnet will determine how many bits must be left in the host portion. Recall that two of the addresses cannot be used, so the usable number of addresses can be calculated as 2n-2.

### Subnetting a /24 Network

| Prefix Length | Subnet Mask | Subnet Mask in Binary (n = network, h = host) | # of subnets | # of hosts per subnet |
| --- | --- | --- | --- | --- |
| /25 | 255.255.255.128 | nnnnnnnn.nnnnnnnn.nnnnnnnn.nhhhhhhh 11111111.11111111.11111111.10000000 | 2 | 126 |
| /26 | 255.255.255.192 | nnnnnnnn.nnnnnnnn.nnnnnnnn.nnhhhhhh 11111111.11111111.11111111.11000000 | 4 | 62 |
| /27 | 255.255.255.224 | nnnnnnnn.nnnnnnnn.nnnnnnnn.nnnhhhhh 11111111.11111111.11111111.11100000 | 8 | 30 |
| /28 | 255.255.255.240 | nnnnnnnn.nnnnnnnn.nnnnnnnn.nnnnhhhh 11111111.11111111.11111111.11110000 | 16 | 14 |
| /29 | 255.255.255.248 | nnnnnnnn.nnnnnnnn.nnnnnnnn.nnnnnhhh 11111111.11111111.11111111.11111000 | 32 | 6 |
| /30 | 255.255.255.252 | nnnnnnnn.nnnnnnnn.nnnnnnnn.nnnnnnhh 11111111.11111111.11111111.11111100 | 64 | 2 |

Network administrators must devise the network addressing scheme to accommodate the maximum number of hosts for each network and the number of subnets. The addressing scheme should allow for growth in both the number of host addresses per subnet and the total number of subnets.

### Network Address

The graphic shows the number of hosts provided when using a 172.16.0.0/22 network. The network portion of the address in binary is: 10101100.00010100.000000. The host portion in binary is: 00.00000000. The host portion consists of 10 host bits therefore 2 to the power of 10 - 2 = 1,022 hosts.

172.16.0.0/2210101100.00010000.00000000. 00000000

10 host bits
210 - 2 = 1,022 hosts

Network portion

Host portion

The corporate headquarters has a DMZ and four branch offices, each needing its own public IPv4 address space. Corporate headquarters needs to make best use of its limited IPv4 address space.

The topology shown in the figure consists of five sites; a corporate office and four branch sites. Each site requires internet connectivity and therefore, five internet connections. This means that the organization requires 10 subnets from the company’s 172.16.0.0/22 public address. The largest subnet requires 40 addresses.

4.1.8 VLSM

IPv4 Address Conservation

Because of the depletion of public IPv4 address space, making the most out of the available host addresses is a primary concern when subnetting IPv4 networks.

Note: The larger IPv6 address allows for much easier address planning and allocation than IPv4 allows. Conserving IPv6 addresses is not an issue. This is one of the driving forces for transitioning to IPv6.

Using traditional subnetting, the same number of addresses is allocated for each subnet. If all the subnets have the same requirements for the number of hosts, or if conserving IPv4 address space is not an issue, these fixed-size address blocks would be efficient. Typically, with public IPv4 addresses, that is not the case.

For example, the topology shown in the figure requires seven subnets, one for each of the four LANs, and one for each of the three connections between the routers.

The diagram shows a network topology consisting of seven subnets. There are four routers, each with an attached LAN and host addressing requirements, and three router-to-router connections requiring 2 hosts each. The R1 router LAN is Building A with 25 hosts; the R2 router LAN is Building B with 20 hosts; the R3 router LAN is Building C with 15 hosts; and the R4 router LAN is Building D with 28 hosts.

R1R2R3R4

Building A

Building B

Building C

Building D

25 hosts

20 hosts

15 hosts

28 hosts

2 hosts

2 hosts

2 hosts

Using traditional subnetting with the given address of 192.168.20.0/24, three bits can be borrowed from the host portion in the last octet to meet the subnet requirement of seven subnets. As shown in the figure, borrowing 3 bits creates 8 subnets and leaves 5 host bits with 30 usable hosts per subnet. This scheme creates the needed subnets and meets the host requirement of the largest LAN.

### Basic Subnet Scheme

The diagram shows the basic subnet scheme for a given address of 192.168.20.0/24 with three bits borrowed for subnetting. Having 3 bits for subnetting results in 2 to the power of 3 = 8 subnets. Having 5 bits for hosts results in 2 to the power of 5 - 2 = 30 host IP addresses per subnet. All four octets are shown in binary followed by the dotted decimal format for the given address and for all the subnets created. The given network address in binary is 11000000.10101000.00010100 (network portion highlighted in gray) .00000000 (host portion highlighted in purple) = 192.168.20.0/24. For the subnets listed below, the first 24 bits are highlighted in gray (network portion), the next three bits are highlighted in blue (subnet portion), and the last five bits are the remaining host bits highlighted in purple. Subnet 0 is 11000000.10101000.00010100.00000000 = 192.168.20.0/27. Subnet 1 is 11000000.10101000.00010100.00100000 = 192.168.20.32/27. Subnet 2 is 11000000.10101000.00010100.01000000 = 192.168.20.64/27. Subnet 3 is 11000000.10101000.00010100.01100000 = 192.168.20.96/27. Subnets 0 - 3 are assigned to building LANs A, B, C, and D. Subnet 4 is 11000000.10101000.00010100.10000000 = 192.168.20.128/27. Subnet 5 is 11000000.10101000.00010100.10100000 = 192.168.20.160/27. Subnet 6 is 11000000.10101000.00010100.11000000 = 192.168.20.192/27. Subnets 4, 5, and 6 are assigned to the site to site WANs. Subnet 7 is 11000000.10101000.00010100.11100000 = 192.168.20.224/27. Subnet 7 is unused/available.

11000000.10101000.00010100.00000000192.168.20.0/246.1100000011000000.10101000.00010100192.168.20.192/275.1010000011000000.10101000.00010100192.168.20.160/274.1000000011000000.10101000.00010100192.168.20.128/273.0110000011000000.10101000.00010100192.168.20.96/272.0100000011000000.10101000.00010100192.168.20.64/271.0010000011000000.10101000.00010100192.168.20.32/270.0000000011000000.10101000.00010100192.168.20.0/277.1110000011000000.10101000.00010100192.168.20.224/27

Building LANs A, B, C, and D

Site to Site WANs

Unused / Available

Network Portion

Host Portion

Subnet portion
2^3 = 8 subnets

Host portion
2^5 – 2 = 30 host IP addresses per subnet

These seven subnets could be assigned to the LAN and WAN networks, as shown in the figure.

### Unused Addresses on WAN Subnets

The graphic shows the unused addresses of four WAN subnets using a /27 subnet mask. All four octets are shown in binary followed by the dotted decimal format for the subnet. The first 24 bits are highlighted in gray (network portion), the next three bits are highlighted in blue, and the last five bits are the remaining host bits highlighted in purple. Subnet 4 is 11000000.10101000.00010100.10000000 = 192.168.20.128/27. Subnet 5 is 11000000.10101000.00010100.10100000 = 192.168.20.160/27. Subnet 6 is 11000000.10101000.00010100.11000000 = 192.168.20.192/27. Having 5 bits for hosts results in 2 to the power of 5 - 2 = 30 host IP addresses per subnet. 30 - 2 = 28; each WAN subnet wastes 28 addresses. 28 x 3 = 84; 84 addresses are unused.

5.1010000011000000.10101000.00010100192.168.20.160/276.1100000011000000.10101000.00010100192.168.20.192/2711000000.10101000.00010100.100000004192.168.20.128/27

Network Portion

Host Portion

Host portion
2^5 – 2 = 30 host IP addresses per subnet

30 – 2 = 28
Each WAN subnet wastes 28 addresses

28 x 3 = 84
84 addresses are unused

Dotted Decimal

Further, this limits future growth by reducing the total number of subnets available. This inefficient use of addresses is characteristic of traditional subnetting. Applying a traditional subnetting scheme to this scenario is not very efficient and is wasteful.

The variable-length subnet mask (VLSM) was developed to avoid wasting addresses by enabling us to subnet a subnet.

VLSM

In all of the previous subnetting examples, the same subnet mask was applied for all the subnets. This means that each subnet has the same number of available host addresses. As illustrated in the left side of the figure, traditional subnetting creates subnets of equal size. Each subnet in a traditional scheme uses the same subnet mask. As shown in the right side of the figure, VLSM allows a network space to be divided into unequal parts. With VLSM, the subnet mask will vary depending on how many bits have been borrowed for a particular subnet, thus the “variable” part of the VLSM.

The graphic shows two pie charts that compare traditional subnetting to VLSM. On the right is a pie chart titled: traditional subnetting creates equal sized subnets. The pie is divided into 8 equal sized slices, each with 30 hosts. On the left is a pie chart titled: subnets of varying sizes. This pie has 7 slices identical to the first pie. The 8th slice has been further divided into 8 smaller slices. Text pointing to 8th slice reads: one subnet was further divided using a /30 subnet mask to create 8 smaller subnets of 2 hosts each.

One subnet was further divided using a /30 subnet mask to create 8 smaller subnets of 2 hosts each.

30 Hosts

30 Hosts

30 Hosts

30 Hosts

30 Hosts

30 Hosts

30 Hosts

30 Hosts

30 Hosts

30 Hosts

30 Hosts

30 Hosts

30 Hosts

30 Hosts

30 Hosts

Traditional Subnetting Creates Equal Sized Subnets

Subnets of Varying Sizes

VLSM is just subnetting a subnet. The same topology used previously is shown in the figure. Again, we will use the 192.168.20.0/24 network and subnet it for seven subnets, one for each of the four LANs, and one for each of the three connections between the routers.

### Basic Subnetting Scheme

The diagram shows the basic subnet scheme for a given address of 192.168.20.0/24 with three bits borrowed for subnetting. All four octets are shown in binary followed by the dotted decimal format for the given address and for all the subnets created. The given network address in binary is 11000000.10101000.00010100 (network portion highlighted in gray) .00000000 (host portion highlighted in purple) = 192.168.20.0/24. For the subnets listed below, the first 24 bits are highlighted in gray (network portion), the next three bits are highlighted in blue (subnet portion), and the last five bits are the remaining host bits highlighted in purple. Subnet 0 is 11000000.10101000.00010100.00000000 = 192.168.20.0/27. Subnet 1 is 11000000.10101000.00010100.00100000 = 192.168.20.32/27. Subnet 2 is 11000000.10101000.00010100.01000000 = 192.168.20.64/27. Subnet 3 is 11000000.10101000.00010100.01100000 = 192.168.20.96/27. Subnets 0 - 3 are assigned to building LANs A, B, C, and D. Subnet 4 is 11000000.10101000.00010100.10000000 = 192.168.20.128/27. Subnet 5 is 11000000.10101000.00010100.10100000 = 192.168.20.160/27. Subnet 6 is 11000000.10101000.00010100.11000000 = 192.168.20.192/27. Subnets 4, 5, and 6 are unused/available. Subnet 7 is 11000000.10101000.00010100.11100000 = 192.168.20.224/27. Subnet 7 will be subnetted further.

11000000.10101000.00010100 .00000000 192.168.20.0/24 0 11000000.10101000.00010100 .000 00000 192.168.20.0/27 1 11000000.10101000.00010100 .001 00000 192.168.20.32/27 2 11000000.10101000.00010100 .010 00000 192.168.20.64/27 3 11000000.10101000.00010100 .011 00000 192.168.20.96/27 4 11000000.10101000.00010100 .100 00000 192.168.20.128/27 5 11000000.10101000.00010100 .101 00000 192.168.20.160/27 6 11000000.10101000.00010100 .110 00000 192.168.20.192/27 7 11000000.10101000.00010100 .111 00000 192.168.20.224/27

Network portion

Host portion

Dotted Decimal

LAN's
A, B, C, D

Unused /
Available

Subnet 7 will be subnetted further.

However, the connections between the routers require only two host addresses per subnet (one host address for each router interface). Currently all subnets have 30 usable host addresses per subnet. To avoid wasting 28 addresses per subnet, VLSM can be used to create smaller subnets for the inter-router connections.

To create smaller subnets for the inter-router links, one of the subnets will be divided. In this example, the last subnet, 192.168.20.224/27, will be further subnetted. The figure shows the last subnet has been subnetted further by using the subnet mask 255.255.255.252 or /30.

### VLSM Subnetting Scheme

The diagram show the VLSM subnetting scheme when the subnet 192.168.20.224/27 is further subnetted by borrowing 3 more bits. For the original subnet, the first 24 bits represent the network portion and are 11000000.10101000.00010100. The next three bits represent the subnet portion and are 111. The last five bits represent the host portion and are 00000. The address in dotted decimal is 192.168.20.224/27. Borrowing 3 additional bits, subnetting a subnet, results in dividing the original subnet into 8 smaller subnets. For the smaller subnets, the first 24 bits are the network portion, the next six bits are the subnet portion, and the last two bits are the remaining host portion. Subnet 7:0 is 11000000.10101000.00010100.11100000 = 192.168.20.224/30. Subnet 7:1 is 11000000.10101000.00010100.11100100 = 192.168.20.228/30. Subnet 7:2 is 11000000.10101000.00010100.11101000 = 192.168.20.232/30. Subnet 7:3 is 11000000.10101000.00010100.11101100 = 192.168.20.236/30. Subnet 7:4 is 11000000.10101000.00010100.11110000 = 192.168.20.240/30. Subnet 7:5 is 11000000.10101000.00010100.11110100 = 192.168.20.244/30. Subnet 7:6 is 11000000.10101000.00010100.11111000 = 192.168.20.248/30. Subnet 7:7 is 11000000.10101000.00010100.11111100 = 192.168.20.252/30. Subnets 7:0, 7:1, and 7:2 are assigned to the WANs and the remaining subnets are unused/available.

7 11000000.10101000.00010100 .111 00000 192.168.20.224/277:0 11000000.10101000.00010100 .111000 00 192.168.20.224/30 7:1 11000000.10101000.00010100 .111001 00 192.168.20.228/30 7:2 11000000.10101000.00010100 .111010 00 192.168.20.232/30 7:3 11000000.10101000.00010100 .111011 00 192.168.20.236/30 7:4 11000000.10101000.00010100 .111100 00 192.168.20.240/30 7:5 11000000.10101000.00010100 .111101 00 192.168.20.244/30 7:6 11000000.10101000.00010100 .111110 00 192.168.20.248/30 7:7 11000000.10101000.00010100 .111111 00 192.168.20.252/30

Network portion

Host portion

Dotted Decimal

WANs

Unused / Available

Subnetting a subnet

3 more bits borrowed from subnet ?

Why /30? Recall that when the number of needed host addresses is known, the formula 2n-2 (where n equals the number of host bits remaining) can be used. To provide two usable addresses, two host bits must be left in the host portion.

Because there are five host bits in the subnetted 192.168.20.224/27 address space, three more bits can be borrowed, leaving two bits in the host portion. The calculations at this point are exactly the same as those used for traditional subnetting. The bits are borrowed, and the subnet ranges are determined. The figure shows how the four /27 subnets have been assigned to the LANs and three of the /30 subnets have been assigned to the inter-router links.

The diagram shows the VLSM subnet assignments for a network topology consisting of four LANs and three WANs. There are four routers, each with an attached LAN and host addressing requirements, and three router-to-router connections requiring 2 hosts each. The R1 router Building A LAN has 25 hosts and is assigned subnet 192.168.20.0/27. The R2 router Building B LAN has 20 hosts and is assigned the subnet 192.168.20.32/27. The R3 router Building C LAN has 15 hosts and is assigned subnet 192.168.20.64/27. The R4 router Building D LAN has 28 hosts and is assigned subnet 192.168.20.96/27. The R1 to R2 connection is assigned subnet 192.168.20.224/30. The R2 to R3 connection is assigned subnet 192.168.20.228/30. The R3 to R4 connection is assigned subnet 192.168.20.232/30.

192.168.20.0/27192.168.20.32/27192.168.20.64/27192.168.20.96/27192.168.20.224/30192.168.20.228/30192.168.20.232/30R1R2R3R4

Building A

Building B

Building C

Building D

25 hosts

20 hosts

15 hosts

28 hosts

2 hosts

2 hosts

2 hosts

This VLSM subnetting scheme reduces the number of addresses per subnet to a size appropriate for the networks that require fewer subnets. Subnetting subnet 7 for inter-router links, allows subnets 4, 5, and 6 to be available for future networks, as well as five additional subnets available for inter-router connections.

Note: When using VLSM, always begin by satisfying the host requirements of the largest subnet. Continue subnetting until the host requirements of the smallest subnet are satisfied.

VLSM Topology Address Assignment

Using the VLSM subnets, the LAN and inter-router networks can be addressed without unnecessary waste.

The figure shows the network address assignments and the IPv4 addresses assigned to each router interface.

The diagram shows the VLSM subnet assignments and interface IP addressing for a network topology consisting of four LANs and three WANs. There are four routers, each with an attached LAN and host addressing requirements, and three router-to-router connections requiring 2 hosts each. The R1 router Building A LAN is connected to the G0/0/0 interface of R1 at 192.168.20.1/27, has 25 hosts, and is assigned subnet 192.168.20.0/27. The R2 router Building B LAN is connected to the G0/0/0 interface of R2 at 192.168.20.33/27, has 20 hosts, and is assigned the subnet 192.168.20.32/27. The R3 router Building C LAN is connected to the G0/0/0 interface of R3 at 192.168.20.65/27, has 15 hosts, and is assigned subnet 192.168.20.64/27. The R4 router Building D LAN is connected to the G0/0/0 interface of R4 at 192.168.20.97/27, has 28 hosts, and is assigned subnet 192.168.20.96/27. The R1 to R2 connection, assigned subnet 192.168.20.224/30, connects the G0/0/1 interface of R1 with address .225 to the G0/0/1 interface of R2 with address .226. The R2 to R3 connection, assigned subnet 192.168.20.228/30, connects the G0/1/0 interface of R2 with address .229 to the G0/0/1 interface of R3 with address .230. The R3 to R4 connection, assigned subnet 192.168.20.232/30, connects the G0/1/0 interface of R2 with address .233 to the G0/0/1 interface of R4 with address .234.

G0/0/1G0/0/1G0/1/0G0/0/1G0/0/1G0/1/0.225.234.233.230 .229.226G0/0/0G0/0/0G0/0/0G0/0/0192.168.20.224/30192.168.20.228/30192.168.20.232/30R1-R2R2-R3R3-R4192.168.20.1/27192.168.20.33/27192.168.20.65/27192.168.20.97/27192.168.20.0/27192.168.20.32/27192.168.20.64/27192.168.20.96/27R1R2R3R4

Building A

Building B

Building C

Building D

25 hosts

20 hosts

15 hosts

28 hosts

2 hosts

2 hosts

2 hosts

Using a common addressing scheme, the first host IPv4 address for each subnet is assigned to the LAN interface of the router. Hosts on each subnet will have a host IPv4 address from the range of host addresses for that subnet and an appropriate mask. Hosts will use the address of the attached router LAN interface as the default gateway address.

The table shows the network addresses and range of host addresses for each network. The default gateway address is displayed for the four LANs.

|  | Network Address | Range of Host Addresses | Default Gateway Address |
| --- | --- | --- | --- |
| Building A | 192.168.20.0/27 | 192.168.20.1/27 to 192.168.20.30/27 | 192.168.20.1/27 |
| Building B | 192.168.20.32/27 | 192.168.20.33/27 to 192.168.20.62/27 | 192.168.20.33/27 |
| Building C | 192.168.20.64/27 | 192.168.20.65/27 to 192.168.20.94/27 | 192.168.20.65/27 |
| Building D | 192.168.20.96/27 | 192.168.20.97/27 to 192.168.20.126/27 | 192.168.20.97/27 |
| R1-R2 | 192.168.20.224/30 | 192.168.20.225/30 to 192.168.20.226/30 |  |
| R2-R3 | 192.168.20.228/30 | 192.168.20.229/30 to 192.168.20.230/30 |  |
| R3-R4 | 192.168.20.232/30 | 192.168.20.233/30 to 192.168.20.234/30 |  |

4.1.9 Structured Design

IPv4 Network Address Planning

Before you start subnetting, you should develop an IPv4 addressing scheme for your entire network. You will need to know how many subnets you need, how many hosts a particular subnet requires, what devices are part of the subnet, which parts of your network use private addresses, and which use public, and many other determining factors. A good addressing scheme allows for growth. A good addressing scheme is also the sign of a good network administrator.

Planning IPv4 network subnets requires you to examine both the needs of an organization’s network usage, and how the subnets will be structured. Performing a network requirement study is the starting point. This means looking at the entire network, both the intranet and the DMZ, and determining how each area will be segmented. The address plan includes determining where address conservation is needed (usually within the DMZ), and where there is more flexibility (usually within the intranet).

Where address conservation is required, the plan should determine how many subnets are needed and how many hosts per subnet. As discussed earlier, this is usually required for public IPv4 address space within the DMZ. This will most likely include using VLSM.

Within the corporate intranet, address conservation is usually less of an issue This is largely due to using private IPv4 addressing, including 10.0.0.0/8, with over 16 million host IPv4 addresses.

For most organizations, private IPv4 addresses allow for more than enough internal (intranet) addresses. For many larger organizations and ISPs, even private IPv4 address space is not large enough to accommodate their internal needs. This is another reason why organizations are transitioning to IPv6.

For intranets that use private IPv4 addresses and DMZs that use public IPv4 addresses, address planning and assignment is important.

Where required, the address plan includes determining the needs of each subnet in terms of size. How many hosts there will be per subnet? The address plan also needs to include how host addresses will be assigned, which hosts will require static IPv4 addresses, and which hosts can use DHCP for obtaining their addressing information. This will also help prevent the duplication of addresses, while allowing for monitoring and managing of addresses for performance and security reasons.

Knowing your IPv4 address requirements will determine the range, or ranges, of host addresses that you implement and help ensure that there are enough addresses to cover your network needs.

Device Address Assignment

Within a network, there are different types of devices that require addresses:

- End user clients - Most networks allocate IPv4 addresses to client devices dynamically, using Dynamic Host Configuration Protocol (DHCP). This reduces the burden on network support staff and virtually eliminates entry errors. With DHCP, addresses are only leased for a period of time, and can be reused when the lease expires. This is an important feature for networks that support transient users and wireless devices. Changing the subnetting scheme means that the DHCP server needs to be reconfigured, and the clients must renew their IPv4 addresses. IPv6 clients can obtain address information using DHCPv6 or SLAAC.

- Servers and peripherals - These should have a predictable static IP address. Use a consistent numbering system for these devices.

- Servers that are accessible from the internet - Servers that need to be publicly available on the internet must have a public IPv4 address, most often accessed using NAT. In some organizations, internal servers (not publicly available) must be made available to the remote users. In most cases, these servers are assigned private addresses internally, and the user is required to create a virtual private network (VPN) connection to access the server. This has the same effect as if the user is accessing the server from a host within the intranet.

- Intermediary devices - These devices are assigned addresses for network management, monitoring, and security. Because we must know how to communicate with intermediary devices, they should have predictable, statically assigned addresses.

- Gateway - Routers and firewall devices have an IP address assigned to each interface which serves as the gateway for the hosts in that network. Typically, the router interface uses either the lowest or highest address in the network.

When developing an IP addressing scheme, it is generally recommended that you have a set pattern of how addresses are allocated to each type of device. This benefits administrators when adding and removing devices, filtering traffic based on IP, as well as simplifying documentation.

4.2 IPV6 Addressing

4.2.1 Need for IPv6

You already know that IPv4 is running out of addresses. That is why you need to learn about IPv6.

IPv6 is designed to be the successor to IPv4. IPv6 has a larger 128-bit address space, providing 340 undecillion (i.e., 340 followed by 36 zeroes) possible addresses. However, IPv6 is more than just larger addresses.

When the IETF began its development of a successor to IPv4, it used this opportunity to fix the limitations of IPv4 and include enhancements. One example is Internet Control Message Protocol version 6 (ICMPv6), which includes address resolution and address autoconfiguration not found in ICMP for IPv4 (ICMPv4).

The depletion of IPv4 address space has been the motivating factor for moving to IPv6. As Africa, Asia and other areas of the world become more connected to the internet, there are not enough IPv4 addresses to accommodate this growth. As shown in the figure, all five RIRs have run out of IPv4 addresses.

IPv4 has a theoretical maximum of 4.3 billion addresses. Private addresses in combination with Network Address Translation (NAT) have been instrumental in slowing the depletion of IPv4 address space. However, NAT is problematic for many applications, creates latency, and has limitations that severely impede peer-to-peer communications.

With the ever-increasing number of mobile devices, mobile providers have been leading the way with the transition to IPv6. The top two mobile providers in the United States report that over 90% of their traffic is over IPv6.

Most top ISPs and content providers such as YouTube, Facebook, and NetFlix, have also made the transition. Many companies like Microsoft, Facebook, and LinkedIn are transitioning to IPv6-only internally. In 2018, broadband ISP Comcast reported a deployment of over 65% and British Sky Broadcasting over 86%.

Internet of Things

The internet of today is significantly different than the internet of past decades. The internet of today is more than email, web pages, and file transfers between computers. The evolving internet is becoming an Internet of Things (IoT). No longer will the only devices accessing the internet be computers, tablets, and smartphones. The sensor-equipped, internet-ready devices of tomorrow will include everything from automobiles and biomedical devices, to household appliances and natural ecosystems.

With an increasing internet population, a limited IPv4 address space, issues with NAT and the IoT, the time has come to begin the transition to IPv6.

IPv4 and IPv6 Coexistence

There is no specific date to move to IPv6. Both IPv4 and IPv6 will coexist in the near future and the transition will take several years. The IETF has created various protocols and tools to help network administrators migrate their networks to IPv6. The migration techniques can be divided into three categories:

Click each button for more information.

Dual Stack

Tunneling

Translation

Dual stack allows IPv4 and IPv6 to coexist on the same network segment. Dual stack devices run both IPv4 and IPv6 protocol stacks simultaneously. Known as native IPv6, this means the customer network has an IPv6 connection to their ISP and is able to access content found on the internet over IPv6.

Note: Tunneling and translation are for transitioning to native IPv6 and should only be used where needed. The goal should be native IPv6 communications from source to destination.

4.2.2 IPv6 Addressing Formats

The first step to learning about IPv6 in networks is to understand the way an IPv6 address is written and formatted. IPv6 addresses are much larger than IPv4 addresses, which is why we are unlikely to run out of them.

IPv6 addresses are 128 bits in length and written as a string of hexadecimal values. Every four bits is represented by a single hexadecimal digit; for a total of 32 hexadecimal values, as shown in the figure. IPv6 addresses are not case-sensitive and can be written in either lowercase or uppercase.

Preferred Format

The previous figure also shows that the preferred format for writing an IPv6 address is x:x:x:x:x:x:x:x, with each “x” consisting of four hexadecimal values. The term octet refers to the eight bits of an IPv4 address. In IPv6, a hextet is the unofficial term used to refer to a segment of 16 bits, or four hexadecimal values. Each “x” is a single hextet which is 16 bits or four hexadecimal digits.

Preferred format means that you write IPv6 address using all 32 hexadecimal digits. It does not necessarily mean that it is the ideal method for representing the IPv6 address. In this module, you will see two rules that help to reduce the number of digits needed to represent an IPv6 address.

These are examples of IPv6 addresses in the preferred format.

2001 : 0db8 : 0000 : 1111 : 0000 : 0000 : 0000: 0200  
  2001 : 0db8 : 0000 : 00a3 : abcd : 0000 : 0000: 1234  
  2001 : 0db8 : 000a : 0001 : c012 : 9aff : fe9a: 19ac  
  2001 : 0db8 : aaaa : 0001 : 0000 : 0000 : 0000: 0000  
  fe80 : 0000 : 0000 : 0000 : 0123 : 4567 : 89ab: cdef  
  fe80 : 0000 : 0000 : 0000 : 0000 : 0000 : 0000: 0001  
  fe80 : 0000 : 0000 : 0000 : c012 : 9aff : fe9a: 19ac  
  fe80 : 0000 : 0000 : 0000 : 0123 : 4567 : 89ab: cdef  
  0000 : 0000 : 0000 : 0000 : 0000 : 0000 : 0000: 0001  
  0000 : 0000 : 0000 : 0000 : 0000 : 0000 : 0000: 0000

The first rule to help reduce the notation of IPv6 addresses is to omit any leading 0s (zeros) in any hextet. Here are four examples of ways to omit leading zeros:

- 01ab can be represented as 1ab

- 09f0 can be represented as 9f0

- 0a00 can be represented as a00

- 00ab can be represented as ab

This rule only applies to leading 0s, NOT to trailing 0s, otherwise the address would be ambiguous. For example, the hextet “abc” could be either “0abc” or “abc0”, but these do not represent the same value.

### Omitting Leading 0s

| Type | Format |
| --- | --- |
| Preferred | 2001 : 0db8 : 0000 : 1111 : 0000 : 0000 : 0000 : 0200 |
| No leading 0s | 2001 : db8 : 0 : 1111 : 0 : 0 : 0 : 200 |
| Preferred | 2001 : 0db8 : 0000 : 00a3 : ab00 : 0ab0 : 00ab : 1234 |
| No leading 0s | 2001 : db8 : 0 : a3 : ab00 : ab0 : ab : 1234 |
| Preferred | 2001 : 0db8 : 000a : 0001 : c012 : 90ff : fe90 : 0001 |
| No leading 0s | 2001 : db8 : a : 1 : c012 : 90ff : fe90 : 1 |
| Preferred | 2001 : 0db8 : aaaa : 0001 : 0000 : 0000 : 0000 : 0000 |
| No leading 0s | 2001 : db8 : aaaa : 1 : 0 : 0 : 0 : 0 |
| Preferred | fe80 : 0000 : 0000 : 0000 : 0123 : 4567 : 89ab : cdef |
| No leading 0s | fe80 : 0 : 0 : 0 : 123 : 4567 : 89ab : cdef |
| Preferred | fe80 : 0000 : 0000 : 0000 : 0000 : 0000 : 0000 : 0001 |
| No leading 0s | fe80 : 0 : 0 : 0 : 0 : 0 : 0 : 1 |
| Preferred | 0000 : 0000 : 0000 : 0000 : 0000 : 0000 : 0000 : 0001 |
| No leading 0s | 0 : 0 : 0 : 0 : 0 : 0 : 0 : 1 |
| Preferred | 0000 : 0000 : 0000 : 0000 : 0000 : 0000 : 0000 : 0000 |
| No leading 0s | 0 : 0 : 0 : 0 : 0 : 0 : 0 : 0 |

12.2.3 Rule 2- Double Colon

The second rule to help reduce the notation of IPv6 addresses is that a double colon (::) can replace any single, contiguous string of one or more 16-bit hextets consisting of all zeros. For example, 2001:db8:cafe:1:0:0:0:1 (leading 0s omitted) could be represented as 2001:db8:cafe:1::1. The double colon (::) is used in place of the three all-0 hextets (0:0:0).

The double colon (::) can only be used once within an address, otherwise there would be more than one possible resulting address. When used with the omitting leading 0s technique, the notation of IPv6 address can often be greatly reduced. This is commonly known as the compressed format.

Here is an example of the incorrect use of the double colon: 2001:db8::abcd::1234.

The double colon is used twice in the example above. Here are the possible expansions of this incorrect compressed format address:

- 2001:db8::abcd:0000:0000:1234

- 2001:db8::abcd:0000:0000:0000:1234

- 2001:db8:0000:abcd::1234

- 2001:db8:0000:0000:abcd::1234

If an address has more than one contiguous string of all-0 hextets, best practice is to use the double colon (::) on the longest string. If the strings are equal, the first string should use the double colon (::).

### Omitting Leading 0s and All 0 Segments

| Type | Format |
| --- | --- |
| Preferred | 2001 : 0db8 : 0000 : 1111 : 0000 : 0000 : 0000 : 0200 |
| Compressed/spaces | 2001 : db8 : 0 : 1111 : : 200 |
| Compressed | 2001:db8:0:1111::200 |
| Preferred | 2001 : 0db8 : 0000 : 0000 : ab00 : 0000 : 0000 : 0000 |
| Compressed/spaces | 2001 : db8 : 0 : 0 : ab00 :: |
| Compressed | 2001:db8:0:0:ab00:: |
| Preferred | 2001 : 0db8 : aaaa : 0001 : 0000 : 0000 : 0000 : 0000 |
| Compressed/spaces | 2001 : db8 : aaaa : 1 :: |
| Compressed | 2001:db8:aaaa:1:: |
| Preferred | fe80 : 0000 : 0000 : 0000 : 0123 : 4567 : 89ab : cdef |
| Compressed/spaces | fe80 : : 123 : 4567 : 89ab : cdef |
| Compressed | fe80::123:4567:89ab:cdef |
| Preferred | fe80 : 0000 : 0000 : 0000 : 0000 : 0000 : 0000 : 0001 |
| Compressed/spaces | fe80 : : 1 |
| Compressed | fe80::1 |
| Preferred | 0000 : 0000 : 0000 : 0000 : 0000 : 0000 : 0000 : 0001 |
| Compressed/spaces | :: 1 |
| Compressed | ::1 |
| Preferred | 0000 : 0000 : 0000 : 0000 : 0000 : 0000 : 0000 : 0000 |
| Compressed/spaces | :: |
| Compressed | :: |

4.2.3 IPV6 Address Types

Unicast, Multicast, Anycast

As with IPv4, there are different types of IPv6 addresses. In fact, there are three broad categories of IPv6 addresses:

- Unicast - An IPv6 unicast address uniquely identifies an interface on an IPv6-enabled device.

- Multicast - An IPv6 multicast address is used to send a single IPv6 packet to multiple destinations.

- Anycast - An IPv6 anycast address is any IPv6 unicast address that can be assigned to multiple devices. A packet sent to an anycast address is routed to the nearest device having that address. Anycast addresses are beyond the scope of this course.

Unlike IPv4, IPv6 does not have a broadcast address. However, there is an IPv6 all-nodes multicast address that essentially gives the same result.

12.3.2 IPv6 Prefix Length

The prefix, or network portion, of an IPv4 address can be identified by a dotted-decimal subnet mask or prefix length (slash notation). For example, an IPv4 address of 192.168.1.10 with dotted-decimal subnet mask 255.255.255.0 is equivalent to 192.168.1.10/24.

In IPv6 it is only called the prefix length. IPv6 does not use the dotted-decimal subnet mask notation. Like IPv4, the prefix length is represented in slash notation and is used to indicate the network portion of an IPv6 address.

The prefix length can range from 0 to 128. The recommended IPv6 prefix length for LANs and most other types of networks is /64, as shown in the figure.

### IPv6 Prefix Length

The graphic shows an IPv6 address divided into a 64-bit prefix and a 64-bit interface ID. The 64-bit prefix is 2001:0db8:000a:0000. The 64-bit interface ID is 0000:0000:0000:0000.

Prefix

Interface ID

0000:0000:0000:0000

2001:0db8:000a:0000

64 bits

64 bits

Example: 2001:db8:a::/64

The prefix or network portion of the address is 64 bits in length, leaving another 64 bits for the interface ID (host portion) of the address.

It is strongly recommended to use a 64-bit Interface ID for most networks. This is because stateless address autoconfiguration (SLAAC) uses 64 bits for the Interface ID. It also makes subnetting easier to create and manage.

Types of IPv6 Unicast Addresses

An IPv6 unicast address uniquely identifies an interface on an IPv6-enabled device. A packet sent to a unicast address is received by the interface which is assigned that address. Similar to IPv4, a source IPv6 address must be a unicast address. The destination IPv6 address can be either a unicast or a multicast address. The figure shows the different types of IPv6 unicast addresses.

- Global Unicast Address (GUA) - This is similar to a public IPv4 address. These are globally unique, internet-routable addresses. GUAs can be configured statically or assigned dynamically.

- Link-local Address (LLA) - This is required for every IPv6-enabled device. LLAs are used to communicate with other devices on the same local link. With IPv6, the term link refers to a subnet. LLAs are confined to a single link. Their uniqueness must only be confirmed on that link because they are not routable beyond the link. In other words, routers will not forward packets with a link-local source or destination address.

A Note About the Unique Local Address

Unique local addresses (range fc00::/7 to fdff::/7) are not yet commonly implemented. Therefore, this module only covers GUA and LLA configuration. However, unique local addresses may eventually be used to address devices that should not be accessible from the outside, such as internal servers and printers.

The IPv6 unique local addresses have some similarity to RFC 1918 private addresses for IPv4, but there are significant differences:

- Unique local addresses are used for local addressing within a site or between a limited number of sites.

- Unique local addresses can be used for devices that will never need to access another network.

- Unique local addresses are not globally routed or translated to a global IPv6 address.

Note: Many sites also use the private nature of RFC 1918 addresses to attempt to secure or hide their network from potential security risks. However, this was never the intended use of these technologies, and the IETF has always recommended that sites take the proper security precautions on their internet-facing router.

IPv6 GUA

IPv6 global unicast addresses (GUAs) are globally unique and routable on the IPv6 internet. These addresses are equivalent to public IPv4 addresses. The Internet Committee for Assigned Names and Numbers (ICANN), the operator for IANA, allocates IPv6 address blocks to the five RIRs. Currently, only GUAs with the first three bits of 001 or 2000::/3 are being assigned, as shown in the figure.

The figure shows the range of values for the first hextet where the first hexadecimal digit for currently available GUAs begins with a 2 or a 3. This is only 1/8th of the total available IPv6 address space, excluding only a very small portion for other types of unicast and multicast addresses.

Note: The 2001:db8::/32 address has been reserved for documentation purposes, including use in examples.

The graphic shows the three parts of a GUA: First is the global routing prefix, then the Subnet ID, then finally the Interface ID. The first three bits of the Global routing prefix are 001. The Range of the first hextext is shown to be from 0010 0000 0000 0000 (2000) to 0011 1111 1111 1111 (3fff)

0010 0000 0000 0000 (2000) 0011 1111 1111 1111 (3fff)001

Interface ID

Subnet ID

Global Routing Prefix

Range of first hextet:

to

The next figure shows the structure and range of a GUA.

### IPv6 Address with a /48 Global Routing Prefix and /64 Prefix

The graphic shows the three parts of a GUA: First is the global routing prefix which is 48 bits in length, then the Subnet ID which is 16 bits in length, then finally the Interface ID which is 64 bits in length. Text under the graphic states A /48 routing prefix + 16 bit Subnet ID = /64 prefix.

48 bits

16 bits

64 bits

A /48 routing prefix + 16 bit Subnet ID = /64 prefix.

64 bits

Interface ID

Subnet ID

Global Routing Prefix

A GUA has three parts:

- Global Routing Prefix

- Subnet ID

- Interface ID

IPv6 GUA Structure

Global Routing Prefix

The global routing prefix is the prefix, or network, portion of the address that is assigned by the provider, such as an ISP, to a customer or site. For example, it is common for ISPs to assign a /48 global routing prefix to its customers. The global routing prefix will usually vary depending on the policies of the ISP.

The previous figure shows a GUA using a /48 global routing prefix. /48 prefixes are a common global routing prefix that is assigned and will be used in most of the examples throughout this course.

For example, the IPv6 address 2001:db8:acad::/48 has a global routing prefix that indicates that the first 48 bits (3 hextets) (2001:db8:acad) is how the ISP knows of this prefix (network). The double colon (::) following the /48 prefix length means the rest of the address contains all 0s. The size of the global routing prefix determines the size of the subnet ID.

Subnet ID

The Subnet ID field is the area between the Global Routing Prefix and the Interface ID. Unlike IPv4 where you must borrow bits from the host portion to create subnets, IPv6 was designed with subnetting in mind. The Subnet ID is used by an organization to identify subnets within its site. The larger the subnet ID, the more subnets available.

Note: Many organizations are receiving a /32 global routing prefix. Using the recommended /64 prefix in order to create a 64-bit Interface ID, leaves a 32 bit Subnet ID. This means an organization with a /32 global routing prefix and a 32-bit Subnet ID will have 4.3 billion subnets, each with 18 quintillion devices per subnet. That is as many subnets as there are public IPv4 addresses!

The IPv6 address in the previous figure has a /48 Global Routing Prefix, which is common among many enterprise networks. This makes it especially easy to examine the different parts of the address. Using a typical /64 prefix length, the first four hextets are for the network portion of the address, with the fourth hextet indicating the Subnet ID. The remaining four hextets are for the Interface ID.

Interface ID

The IPv6 interface ID is equivalent to the host portion of an IPv4 address. The term Interface ID is used because a single host may have multiple interfaces, each having one or more IPv6 addresses. The figure shows an example of the structure of an IPv6 GUA. It is strongly recommended that in most cases /64 subnets should be used, which creates a 64-bit interface ID. A 64-bit interface ID allows for 18 quintillion devices or hosts per subnet.

A /64 subnet or prefix (Global Routing Prefix + Subnet ID) leaves 64 bits for the interface ID. This is recommended to allow SLAAC-enabled devices to create their own 64-bit interface ID. It also makes developing an IPv6 addressing plan simple and effective.

Note: Unlike IPv4, in IPv6, the all-0s and all-1s host addresses can be assigned to a device. The all-1s address can be used because broadcast addresses are not used within IPv6. The all-0s address can also be used, but is reserved as a Subnet-Router anycast address, and should be assigned only to routers.

IPv6 LLA

An IPv6 link-local address (LLA) enables a device to communicate with other IPv6-enabled devices on the same link and only on that link (subnet). Packets with a source or destination LLA cannot be routed beyond the link from which the packet originated.

The GUA is not a requirement. However, every IPv6-enabled network interface must have an LLA.

If an LLA is not configured manually on an interface, the device will automatically create its own without communicating with a DHCP server. IPv6-enabled hosts create an IPv6 LLA even if the device has not been assigned a global unicast IPv6 address. This allows IPv6-enabled devices to communicate with other IPv6-enabled devices on the same subnet. This includes communication with the default gateway (router).

IPv6 LLAs are in the fe80::/10 range. The /10 indicates that the first 10 bits are 1111 1110 10xx xxxx. The first hextet has a range of 1111 1110 1000 0000 (fe80) to 1111 1110 1011 1111 (febf).

The figure shows an example of communication using IPv6 LLAs. The PC is able to communicate directly with the printer using the LLAs.

There are two ways that a device can obtain an LLA:

- Statically - This means the device has been manually configured.

- Dynamically - This means the device creates its own interface ID by using randomly generated values or using the Extended Unique Identifier (EUI) method, which uses the client MAC address along with additional bits.

4.2.4 GUA and LLA Static Configuration

Static GUA Configuration on a Router

As you learned in the previous topic, IPv6 GUAs are the same as public IPv4 addresses. They are globally unique and routable on the IPv6 internet. An IPv6 LLA lets two IPv6-enabled devices communicate with each other on the same link (subnet). It is easy to statically configure IPv6 GUAs and LLAs on routers to help you create an IPv6 network. This topic teaches you how to do just that!

Most IPv6 configuration and verification commands in the Cisco IOS are similar to their IPv4 counterparts. In many cases, the only difference is the use of ipv6 in place of ip within the commands.

For example, the Cisco IOS command to configure an IPv4 address on an interface is ip address ip-address subnet-mask. In contrast, the command to configure an IPv6 GUA on an interface is ipv6 address ipv6-address/prefix-length.

Notice that there is no space between ipv6-address and prefix-length.

The example configuration uses the topology shown in the figure and these IPv6 subnets:

- 2001:db8:acad:1::/64

- 2001:db8:acad:2::/64

- 2001:db8:acad:3::/64

### Example Topology

The graphic shows two PCs, PC1 and PC2. PC1 is connected to a switch and has the IPv6 address 2001:db8:acad:1::10/64. PC2 is connected to a switch and has the IPv6 address 2001:db8:acad:2::10/64. The two switches are connected to a router, R1. PC1 is connected through the switch to R1s G0/0/0 interface which has IPv6 address 2001:db8:acad:1::1/64. PC2 is connected through the switch to R1s G0/0/1 interface which has IPv6 address 2001:db8:acad:2::1/64. R1 connects to the cloud through its S0/1/0 interface which has IPv6 address 2001:db8:acad:3::1/64.

R1::10::10S0/1/0::1PC1PC22001:db8.acad:2::/642001:db8.acad:1::/642001:db8.acad:3::/64G0/0/0::1G0/0/1::1R1

The example shows the commands required to configure the IPv6 GUA on GigabitEthernet 0/0/0, GigabitEthernet 0/0/1, and the Serial 0/1/0 interface of R1.

### IPv6 GUA Configuration on Router R1

R1(config)# interface gigabitethernet 0/0/0

R1(config-if)# ipv6 address 2001:db8:acad:1::1/64

R1(config-if)# no shutdown

R1(config-if)# exit

R1(config)# interface gigabitethernet 0/0/1

R1(config-if)# ipv6 address 2001:db8:acad:2::1/64

R1(config-if)# no shutdown

R1(config-if)# exit

R1(config)# interface serial 0/1/0

R1(config-if)# ipv6 address 2001:db8:acad:3::1/64

R1(config-if)# no shutdown

12.4.2 Static GUA Configuration on a Windows Host

Manually configuring the IPv6 address on a host is similar to configuring an IPv4 address.

As shown in the figure, the default gateway address configured for PC1 is 2001:db8:acad:1::1. This is the GUA of the R1 GigabitEthernet interface on the same network. Alternatively, the default gateway address can be configured to match the LLA of the GigabitEthernet interface. Using the LLA of the router as the default gateway address is considered best practice. Either configuration will work.

The graphic shows the Windows Internet Protocol Version 6 (TCP/IPv6) Prosperities window. The Use the following IPv6 address button is selected. The IPv6 address is 2001:db8:acad:1::1. The Subnet prefix length is 64. The Default gateway is 2001:db8:acad:1::1. The Use the following DNS server address button is selected.

Just as with IPv4, configuring static addresses on clients does not scale to larger environments. For this reason, most network administrators in an IPv6 network will enable dynamic assignment of IPv6 addresses.

There are two ways in which a device can obtain an IPv6 GUA automatically:

- Stateless Address Autoconfiguration (SLAAC)

- Stateful DHCPv6

SLAAC and DHCPv6 are covered in the next topic.

Note: When DHCPv6 or SLAAC is used, the LLA of the router will automatically be specified as the default gateway address.

Configuring the LLA manually lets you create an address that is recognizable and easier to remember. Typically, it is only necessary to create recognizable LLAs on routers. This is beneficial because router LLAs are used as default gateway addresses and in routing advertisement messages.

LLAs can be configured manually using the ipv6 address ipv6-link-local-address link-local command. When an address begins with this hextet within the range of fe80 to febf, the link-local parameter must follow the address.

### Example Topology with LLAs

The graphic shows two PCs, PC1 and PC2. PC1 is connected to a switch and has the IPv6 address 2001:db8:acad:1::10/64. PC2 is connected to a switch and has the IPv6 address 2001:db8:acad:2::10/64. The two switches are connected to a router, R1. PC1 is connected through the switch to R1s G0/0/0 interface which has IPv6 address 2001:db8:acad:1::1/64 and the LLA address of fe80::1:1. PC2 is connected through the switch to R1s G0/0/1 interface which has IPv6 address 2001:db8:acad:2::1/64 and the LLA address of fe80::2:1. R1 connects to the cloud through its S0/1/0 interface which has IPv6 address 2001:db8:acad:3::1/64 and the LLA address of fe80::3:1.

R1::10::10S0/1/0::1fe80::3:1PC12001:db8.acad:2::/642001:db8.acad:1::/642001:db8.acad:3::/64G0/0/0::1fe80::1:1G0/0/1::1fe80::2:1PC2R1

The example shows the configuration of an LLA on router R1.

R1(config)# interface gigabitethernet 0/0/0

R1(config-if)# ipv6 address fe80::1:1 link-local

R1(config-if)# exit

R1(config)# interface gigabitethernet 0/0/1

R1(config-if)# ipv6 address fe80::2:1 link-local

R1(config-if)# exit

R1(config)# interface serial 0/1/0

R1(config-if)# ipv6 address fe80::3:1 link-local

R1(config-if)# exit

Statically configured LLAs are used to make them more easily recognizable as belonging to router R1. In this example, all the interfaces of router R1 have been configured with an LLA that begins with fe80::n:1.

Note: The exact same LLA could be configured on each link as long as it is unique on that link. This is because LLAs only have to be unique on that link. However, common practice is to create a different LLA on each interface of the router to make it easy to identify the router and the specific interface.

4.2.5 Dynamic Addressing for IPV6 GUAs

RS and RA Messages

If you do not want to statically configure IPv6 GUAs, no need to worry. Most devices obtain their IPv6 GUAs dynamically. This topic explains how this process works using Router Advertisement (RA) and Router Solicitation (RS) messages. This topic gets rather technical, but when you understand the difference between the three methods that a router advertisement can use, as well as how the EUI-64 process for creating an interface ID differs from a randomly generated process, you will have made a huge leap in your IPv6 expertise!

For the GUA, a device obtains the address dynamically through Internet Control Message Protocol version 6 (ICMPv6) messages. IPv6 routers periodically send out ICMPv6 RA messages, every 200 seconds, to all IPv6-enabled devices on the network. An RA message will also be sent in response to a host sending an ICMPv6 RS message, which is a request for an RA message. Both messages are shown in the figure.

### ICMPv6 RS and RA Messages

RA messages are on IPv6 router Ethernet interfaces. The router must be enabled for IPv6 routing, which is not enabled by default. To enable a router as an IPv6 router, the ipv6 unicast-routing global configuration command must be used.

The ICMPv6 RA message is a suggestion to a device on how to obtain an IPv6 GUA. The ultimate decision is up to the device operating system. The ICMPv6 RA message includes the following:

- Network prefix and prefix length - This tells the device which network it belongs to.

- Default gateway address - This is an IPv6 LLA, the source IPv6 address of the RA message.

- DNS addresses and domain name - These are the addresses of DNS servers and a domain name.

There are three methods for RA messages:

- Method 1: SLAAC - “I have everything you need including the prefix, prefix length, and default gateway address.”

- Method 2: SLAAC with a stateless DHCPv6 server - “Here is my information but you need to get other information such as DNS addresses from a stateless DHCPv6 server.”

- Method 3: Stateful DHCPv6 (no SLAAC) - “I can give you your default gateway address. You need to ask a stateful DHCPv6 server for all your other information.”

Method 1 - SLAAC

SLAAC is a method that allows a device to create its own GUA without the services of DHCPv6. Using SLAAC, devices rely on the ICMPv6 RA messages of the local router to obtain the necessary information.

By default, the RA message suggests that the receiving device use the information in the RA message to create its own IPv6 GUA and all other necessary information. The services of a DHCPv6 server are not required.

SLAAC is stateless, which means there is no central server (for example, a stateful DHCPv6 server) allocating GUAs and keeping a list of devices and their addresses. With SLAAC, the client device uses the information in the RA message to create its own GUA. As shown in the figure, the two parts of the address are created as follows:

- Prefix - This is advertised in the RA message.

- Interface ID - This uses the EUI-64 process or by generating a random 64-bit number, depending on the device operating system.

Method 2 - SLAAC and Stateless DHCPv6

A router interface can be configured to send a router advertisement using SLAAC and stateless DHCPv6.

- SLAAC to create its own IPv6 GUA

- The router LLA, which is the RA source IPv6 address, as the default gateway address

- A stateless DHCPv6 server to obtain other information such as a DNS server address and a domain name

Note: A stateless DHCPv6 server distributes DNS server addresses and domain names. It does not allocate GUAs.

Method 3 - Stateful DHCPv6

A router interface can be configured to send an RA using stateful DHCPv6 only.

Stateful DHCPv6 is similar to DHCP for IPv4. A device can automatically receive its addressing information including a GUA, prefix length, and the addresses of DNS servers from a stateful DHCPv6 server.

As shown in the figure, with this method, the RA message suggests devices use the following:

- The router LLA, which is the RA source IPv6 address, for the default gateway address.

- A stateful DHCPv6 server to obtain a GUA, DNS server address, domain name and other necessary information.

Note: The default gateway address can only be obtained dynamically from the RA message. The stateless or stateful DHCPv6 server does not provide the default gateway address.

EUI-64 Process vs. Randomly Generated

When the RA message is either SLAAC or SLAAC with stateless DHCPv6, the client must generate its own interface ID. The client knows the prefix portion of the address from the RA message, but must create its own interface ID. The interface ID can be created using the EUI-64 process or a randomly generated 64-bit number, as shown in the figure.

EUI-64 Process

IEEE defined the Extended Unique Identifier (EUI) or modified EUI-64 process. This process uses the 48-bit Ethernet MAC address of a client, and inserts another 16 bits in the middle of the 48-bit MAC address to create a 64-bit interface ID.

Ethernet MAC addresses are usually represented in hexadecimal and are made up of two parts:

- Organizationally Unique Identifier (OUI) - The OUI is a 24-bit (6 hexadecimal digits) vendor code assigned by IEEE.

- Device Identifier - The device identifier is a unique 24-bit (6 hexadecimal digits) value within a common OUI.

An EUI-64 Interface ID is represented in binary and is made up of three parts:

- 24-bit OUI from the client MAC address, but the 7th bit (the Universally/Locally (U/L) bit) is reversed. This means that if the 7th bit is a 0, it becomes a 1, and vice versa.

- The inserted 16-bit value fffe (in hexadecimal).

- 24-bit Device Identifier from the client MAC address.

The advantage of EUI-64 is that the Ethernet MAC address can be used to determine the interface ID. It also allows network administrators to easily track an IPv6 address to an end-device using the unique MAC address. However, this has caused privacy concerns among many users who worried that their packets could be traced to the actual physical computer. Due to these concerns, a randomly generated interface ID may be used instead.

### EUI-64 Generated Interface ID

C:\> ipconfig

Windows IP Configuration

Ethernet adapter Local Area Connection:

Connection-specific DNS Suffix  . :

IPv6 Address. . . . . . . . . . . : 2001:db8:acad:1:fc99:47ff:fe75:cee0

Link-local IPv6 Address . . . . . : fe80::fc99:47ff:fe75:cee0

Default Gateway . . . . . . . . . : fe80::1

C:\>

12.5.7 Randomly Generated Interface IDs

Depending upon the operating system, a device may use a randomly generated interface ID instead of using the MAC address and the EUI-64 process. Beginning with Windows Vista, Windows uses a randomly generated interface ID instead of one created with EUI-64. Windows XP and previous Windows operating systems used EUI-64.

After the interface ID is established, either through the EUI-64 process or through random generation, it can be combined with an IPv6 prefix in the RA message to create a GUA, as shown in the figure.

### Random 64-Bit Generated Interface ID

C:\> ipconfig

Windows IP Configuration

Ethernet adapter Local Area Connection:

Connection-specific DNS Suffix  . :

IPv6 Address. . . . . . . . . . . : 2001:db8:acad:1:50a5:8a35:a5bb:66e1

Link-local IPv6 Address . . . . . : fe80::50a5:8a35:a5bb:66e1

Default Gateway . . . . . . . . . : fe80::1

C:\>

Note: To ensure the uniqueness of any IPv6 unicast address, the client may use a process known as Duplicate Address Detection (DAD). This is similar to an ARP request for its own address. If there is no reply, then the address is unique.

4.2.6 Dynamic Addressing for IPV6 LLAs

Dynamic LLAs

All IPv6 devices must have an IPv6 LLA. Like IPv6 GUAs, you can also create LLAs dynamically. Regardless of how you create your LLAs (and your GUAs), it is important that you verify all IPv6 address configuration. This topic explains dynamically generated LLAs and IPv6 configuration verification.

Dynamic LLAs on Windows

Operating systems, such as Windows, will typically use the same method for both a SLAAC-created GUA and a dynamically assigned LLA. See the highlighted areas in the following examples that were shown previously.

### EUI-64 Generated Interface ID

C:\> ipconfig

Windows IP Configuration

Ethernet adapter Local Area Connection:

Connection-specific DNS Suffix . :

IPv6 Address. . . . . . . . . . . : 2001:db8:acad:1:fc99:47ff:fe75:cee0

Link-local IPv6 Address . . . . . : fe80::fc99:47ff:fe75:cee0

Default Gateway . . . . . . . . . : fe80::1

C:\>

### Random 64-Bit Generated Interface ID

C:\> ipconfig

Windows IP Configuration

Ethernet adapter Local Area Connection:

Connection-specific DNS Suffix  . :

IPv6 Address. . . . . . . . . . . : 2001:db8:acad:1:50a5:8a35:a5bb:66e1

Link-local IPv6 Address . . . . . : fe80::50a5:8a35:a5bb:66e1

Default Gateway . . . . . . . . . : fe80::1

C:\>

Dynamic LLAs on Cisco Routers

Cisco routers automatically create an IPv6 LLA whenever a GUA is assigned to the interface. By default, Cisco IOS routers use EUI-64 to generate the interface ID for all LLAs on IPv6 interfaces. For serial interfaces, the router will use the MAC address of an Ethernet interface. Recall that an LLA must be unique only on that link or network. However, a drawback to using the dynamically assigned LLA is its long interface ID, which makes it challenging to identify and remember assigned addresses. The example displays the MAC address on the GigabitEthernet 0/0/0 interface of router R1. This address is used to dynamically create the LLA on the same interface, and also for the Serial 0/1/0 interface.

To make it easier to recognize and remember these addresses on routers, it is common to statically configure IPv6 LLAs on routers.

### IPv6 LLA Using EUI-64 on Router R1

R1# show interface gigabitEthernet 0/0/0

GigabitEthernet0/0/0 is up, line protocol is up

Hardware is ISR4221-2x1GE, address is 7079.b392.3640 (bia 7079.b392.3640)

(Output omitted)

R1# show ipv6 interface brief

GigabitEthernet0/0/0   [up/up]

FE80::7279:B3FF:FE92:3640

2001:DB8:ACAD:1::1

GigabitEthernet0/0/1   [up/up]

FE80::7279:B3FF:FE92:3641

2001:DB8:ACAD:2::1

Serial0/1/0            [up/up]

FE80::7279:B3FF:FE92:3640

2001:DB8:ACAD:3::1

Serial0/1/1            [down/down]

unassigned

R1#

Verify IPv6 Address Configuration

The show ipv6 interface brief command displays the IPv6 address of the Ethernet interfaces. EUI-64 uses this MAC address to generate the interface ID for the LLA. Additionally, the show ipv6 interface brief command displays abbreviated output for each of the interfaces. The [up/up] output on the same line as the interface indicates the Layer 1/Layer 2 interface state. This is the same as the Status and Protocol columns in the equivalent IPv4 command.

Notice that each interface has two IPv6 addresses. The second address for each interface is the GUA that was configured. The first address, the one that begins with fe80, is the link-local unicast address for the interface. Recall that the LLA is automatically added to the interface when a GUA is assigned.

Also, notice that the R1 Serial 0/1/0 LLA is the same as its GigabitEthernet 0/0/0 interface. Serial interfaces do not have Ethernet MAC addresses, so Cisco IOS uses the MAC address of the first available Ethernet interface. This is possible because link-local interfaces only have to be unique on that link.

The show ipv6 interface brief Command on R1

R1# show ipv6 interface brief

GigabitEthernet0/0/0   [up/up]

FE80::7279:b3ff:fe92:3640

2001:DB8:ACAD:1::1

GigabitEthernet0/0/1   [up/up]

FE80::8320:e4ff:fe79:2830

2001:DB8:ACAD:2::1

Serial0/1/0            [up/up]

FE80::7279:b3ff:fe92:3640

2001:DB8:ACAD:3::1

Serial0/1/1            [down/down]

unassigned

R1#

4.2.7 IPV6 Multicast Addresses

Assigned IPv6 Multicast Addresses

Earlier in this module, you learned that there are three broad categories of IPv6 addresses: unicast, anycast, and multicast. This topic goes into more detail about multicast addresses.

IPv6 multicast addresses are similar to IPv4 multicast addresses. Recall that a multicast address is used to send a single packet to one or more destinations (multicast group). IPv6 multicast addresses have the prefix ff00::/8.

Note: Multicast addresses can only be destination addresses and not source addresses.

There are two types of IPv6 multicast addresses:

- Well-known multicast addresses

- Solicited node multicast addresses

Well-Known IPv6 Multicast Addresses

Well-known IPv6 multicast addresses are assigned. Assigned multicast addresses are reserved multicast addresses for predefined groups of devices. An assigned multicast address is a single address used to reach a group of devices running a common protocol or service. Assigned multicast addresses are used in context with specific protocols such as DHCPv6.

These are two common IPv6 assigned multicast groups:

- ff02::1 All-nodes multicast group - This is a multicast group that all IPv6-enabled devices join. A packet sent to this group is received and processed by all IPv6 interfaces on the link or network. This has the same effect as a broadcast address in IPv4. The figure shows an example of communication using the all-nodes multicast address. An IPv6 router sends ICMPv6 RA messages to the all-node multicast group.

- ff02::2 All-routers multicast group - This is a multicast group that all IPv6 routers join. A router becomes a member of this group when it is enabled as an IPv6 router with the ipv6 unicast-routing global configuration command. A packet sent to this group is received and processed by all IPv6 routers on the link or network.

.

Solicited-Node IPv6 Multicast Addresses

A solicited-node multicast address is similar to the all-nodes multicast address. The advantage of a solicited-node multicast address is that it is mapped to a special Ethernet multicast address. This allows the Ethernet NIC to filter the frame by examining the destination MAC address without sending it to the IPv6 process to see if the device is the intended target of the IPv6 packet.

4.2.8 Subnet an IPV6 Network

Subnet Using the Subnet ID

The introduction to this module mentioned subnetting an IPv6 network. It also said that you might discover that it is a bit easier than subnetting an IPv4 network. You are about to find out!

Recall that with IPv4, we must borrow bits from the host portion to create subnets. This is because subnetting was an afterthought with IPv4. However, IPv6 was designed with subnetting in mind. A separate subnet ID field in the IPv6 GUA is used to create subnets. As shown in the figure, the subnet ID field is the area between the Global Routing Prefix and the interface ID.

### GUA with a 16-bit Subnet ID

The benefit of a 128-bit address is that it can support more than enough subnets and hosts per subnet, for each network. Address conservation is not an issue. For example, if the global routing prefix is a /48, and using a typical 64 bits for the interface ID, this will create a 16-bit subnet ID:

- 16-bit subnet ID - Creates up to 65,536 subnets.

- 64-bit interface ID - Supports up to 18 quintillion host IPv6 addresses per subnet (i.e., 18,000,000,000,000,000,000).

Note: Subnetting into the 64-bit interface ID (or host portion) is also possible but it is rarely required.

IPv6 subnetting is also easier to implement than IPv4, because there is no conversion to binary required. To determine the next available subnet, just count up in hexadecimal.

IPv6 Subnetting Example

For example, assume an organization has been assigned the 2001:db8:acad::/48 global routing prefix with a 16 bit subnet ID. This would allow the organization to create 65,536 /64 subnets, as shown in the figure. Notice how the global routing prefix is the same for all subnets. Only the subnet ID hextet is incremented in hexadecimal for each subnet.

IPv6 Subnet Allocation

With over 65,536 subnets to choose from, the task of the network administrator becomes one of designing a logical scheme to address the network.

### IPv6 Address Configuration on Router R1

R1(config)# interface gigabitethernet 0/0/0

R1(config-if)# ipv6 address 2001:db8:acad:1::1/64

R1(config-if)# no shutdown

R1(config-if)# exit

R1(config)# interface gigabitethernet 0/0/1

R1(config-if)# ipv6 address 2001:db8:acad:2::1/64

R1(config-if)# no shutdown

R1(config-if)# exit

R1(config)# interface serial 0/1/0

R1(config-if)# ipv6 address 2001:db8:acad:3::1/64

R1(config-if)# no shutdown

4.3 ICMP

4.3.1 ICMPv4 and ICMPv6 Messages

In this topic, you will learn about the different types of Internet Control Message Protocols (ICMPs), and the tools that are used to send them.

Although IP is only a best-effort protocol, the TCP/IP suite does provide for error messages and informational messages when communicating with another IP device. These messages are sent using the services of ICMP. The purpose of these messages is to provide feedback about issues related to the processing of IP packets under certain conditions, not to make IP reliable. ICMP messages are not required and are often not allowed within a network for security reasons.

ICMP is available for both IPv4 and IPv6. ICMPv4 is the messaging protocol for IPv4. ICMPv6 provides these same services for IPv6 but includes additional functionality. In this course, the term ICMP will be used when referring to both ICMPv4 and ICMPv6.

The types of ICMP messages, and the reasons why they are sent, are extensive. The ICMP messages common to both ICMPv4 and ICMPv6 and discussed in this module include:

- Host reachability

- Destination or Service Unreachable

- Time exceeded

Host Reachability

An ICMP Echo Message can be used to test the reachability of a host on an IP network. The local host sends an ICMP Echo Request to a host. If the host is available, the destination host responds with an Echo Reply. In the figure, click the Play button to see an animation of the ICMP Echo Request/Echo Reply. This use of the ICMP Echo messages is the basis of the ping utility.

animation of host 1 sending a ping ICMP echo request to host 2 and the ICMP echo reply from host 2 back to host 1

Yes, I am here.

ping 192.168.30.1

Destination or Service Unreachable

When a host or gateway receives a packet that it cannot deliver, it can use an ICMP Destination Unreachable message to notify the source that the destination or service is unreachable. The message will include a code that indicates why the packet could not be delivered.

Some of the Destination Unreachable codes for ICMPv4 are as follows:

- 0 - Net unreachable

- 1 - Host unreachable

- 2 - Protocol unreachable

- 3 - Port unreachable

Some of the Destination Unreachable codes for ICMPv6 are as follows:

- 0 - No route to destination

- 1 - Communication with the destination is administratively prohibited (e.g., firewall)

- 2 - Beyond scope of the source address

- 3 - Address unreachable

- 4 - Port unreachable

Note: ICMPv6 has similar but slightly different codes for Destination Unreachable messages.

Time Exceeded

An ICMPv4 Time Exceeded message is used by a router to indicate that a packet cannot be forwarded because the Time to Live (TTL) field of the packet was decremented to 0. If a router receives a packet and decrements the TTL field in the IPv4 packet to zero, it discards the packet and sends a Time Exceeded message to the source host.

ICMPv6 also sends a Time Exceeded message if the router cannot forward an IPv6 packet because the packet has expired. Instead of the IPv4 TTL field, ICMPv6 uses the IPv6 Hop Limit field to determine if the packet has expired.

Note: Time Exceeded messages are used by the traceroute tool.

ICMPv6 Messages

The informational and error messages found in ICMPv6 are very similar to the control and error messages implemented by ICMPv4. However, ICMPv6 has new features and improved functionality not found in ICMPv4. ICMPv6 messages are encapsulated in IPv6.

ICMPv6 includes four new protocols as part of the Neighbor Discovery Protocol (ND or NDP).

Messaging between an IPv6 router and an IPv6 device, including dynamic address allocation are as follows:

- Router Solicitation (RS) message

- Router Advertisement (RA) message

Messaging between IPv6 devices, including duplicate address detection and address resolution are as follows:

- Neighbor Solicitation (NS) message

- Neighbor Advertisement (NA) message

Note: ICMPv6 ND also includes the redirect message, which has a similar function to the redirect message used in ICMPv4.

4.3.2 Ping and Trace route Tests

Ping - Test Connectivity

In the previous topic, you were introduced to the ping and traceroute (tracert) tools. In this topic, you will learn about the situations in which each tool is used, and how to use them. Ping is an IPv4 and IPv6 testing utility that uses ICMP echo request and echo reply messages to test connectivity between hosts.

To test connectivity to another host on a network, an echo request is sent to the host address using the ping command. If the host at the specified address receives the echo request, it responds with an echo reply. As each echo reply is received, ping provides feedback on the time between when the request was sent and when the reply was received. This can be a measure of network performance.

Ping has a timeout value for the reply. If a reply is not received within the timeout, ping provides a message indicating that a response was not received. This may indicate that there is a problem, but could also indicate that security features blocking ping messages have been enabled on the network. It is common for the first ping to timeout if address resolution (ARP or ND) needs to be performed before sending the ICMP Echo Request.

After all the requests are sent, the ping utility provides a summary that includes the success rate and average round-trip time to the destination.

Type of connectivity tests performed with ping include the following:

- Pinging the local loopback

- Pinging the default gateway

- Pinging the remote host

Ping the Loopback

Ping can be used to test the internal configuration of IPv4 or IPv6 on the local host. To perform this test, ping the local loopback address of 127.0.0.1 for IPv4 (::1 for IPv6).

A response from 127.0.0.1 for IPv4, or ::1 for IPv6, indicates that IP is properly installed on the host. This response comes from the network layer. This response is not, however, an indication that the addresses, masks, or gateways are properly configured. Nor does it indicate anything about the status of the lower layer of the network stack. This simply tests IP down through the network layer of IP. An error message indicates that TCP/IP is not operational on the host.

shows the Ethernet properties dialogue box shows that Internet Protocol Version 4 (TCP/IPv4) is installed and active which is proved with a ping to 127.0.0.1

C:\>ping 127.0.0.1

- Pinging the local host confirms that TCP/IP is installed and working on the local host.

- Pinging 127.0.0.1 causes a device to ping itself.

Ping the Default Gateway

You can also use ping to test the ability of a host to communicate on the local network. This is generally done by pinging the IP address of the default gateway of the host. A successful ping to the default gateway indicates that the host and the router interface serving as the default gateway are both operational on the local network.

For this test, the default gateway address is most often used because the router is normally always operational. If the default gateway address does not respond, a ping can be sent to the IP address of another host on the local network that is known to be operational.

If either the default gateway or another host responds, then the local host can successfully communicate over the local network. If the default gateway does not respond but another host does, this could indicate a problem with the router interface serving as the default gateway.

One possibility is that the wrong default gateway address has been configured on the host. Another possibility is that the router interface may be fully operational but have security applied to it that prevents it from processing or responding to ping requests.

The graphic shows the Ethernet properties dialogue box configured with a static IP address, subnet mask, and default gateway. The topology shows the PC sending an echo request to the router default gateway and the routers echo response reply.

ECHO REQUEST

ECHO REPLY

10.0.0.1
255.255.255.0

10.0.0.254
255.255.255.0

G0/0/0

The host pings its default gateway, sending an ICMP echo request. The default gateway sends an echo reply confirming connectivity.

Ping a Remote Host

Ping can also be used to test the ability of a local host to communicate across an internetwork. The local host can ping an operational IPv4 host of a remote network, as shown in the figure. The router uses its IP routing table to forward the packets.

If this ping is successful, the operation of a large piece of the internetwork can be verified. A successful ping across the internetwork confirms communication on the local network, the operation of the router serving as the default gateway, and the operation of all other routers that might be in the path between the local network and the network of the remote host.

Additionally, the functionality of the remote host can be verified. If the remote host could not communicate outside of its local network, it would not have responded.

Note: Many network administrators limit or prohibit the entry of ICMP messages into the corporate network; therefore, the lack of a ping response could be due to security restrictions.

Traceroute - Test the Path

Ping is used to test connectivity between two hosts but does not provide information about the details of devices between the hosts. Traceroute (tracert) is a utility that generates a list of hops that were successfully reached along the path. This list can provide important verification and troubleshooting information. If the data reaches the destination, then the trace lists the interface of every router in the path between the hosts. If the data fails at some hop along the way, the address of the last router that responded to the trace can provide an indication of where the problem or security restrictions are found.

Round Trip Time (RTT)

Using traceroute provides round-trip time for each hop along the path and indicates if a hop fails to respond. The round-trip time is the time a packet takes to reach the remote host and for the response from the host to return. An asterisk (*) is used to indicate a lost or unreplied packet.

This information can be used to locate a problematic router in the path or may indicate that the router is configured not to reply. If the display shows high response times or data losses from a particular hop, this is an indication that the resources of the router or its connections may be stressed.

Top of Form

1.1.1.1.12.5%Class B25%

Class A

Total Networks: 128
Total Hosts/Net: 16,777,214

Class B

Total Networks: 16,384
Total Hosts/Net: 65,534

Class C

Total Networks: 2,097,152
Total Hosts/Net: 254